# SIMATS ENGINEERING ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA0713

# COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Dr.V.Parthipan

Annexure A

# Debugging & Optimisation Task

## Code of Conduct:

I S HARIPRIYA(192412448) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

## priya

## Signature of the student:

+SECTION 1 — Cyclic Redundancy Check (CRC) using X³ + 1

## Introduction and Theory

Cyclic Redundancy Check (CRC) is a very powerful and easy-to-implement error-detection technique used in nearly every modern data-link layer protocol — Ethernet (CRC-32), Wi-Fi, Bluetooth, HDLC, USB, and even in ZIP and PNG file formats. The core idea is to treat any block of k message bits as the coefficients of a polynomial M(x) of degree k–1, and then divide it (modulo-2, i.e. bitwise XOR without carries) by an agreed-upon generator polynomial G(x) of degree r. The r-bit remainder R(x) produced by this division is appended to the message and transmitted as the Frame Check Sequence (FCS).

Why CRC is preferred over a simple checksum or parity bit:

    - A single parity bit detects only an odd number of bit errors.

    - A checksum can miss reordered or paired errors.

    - A well-chosen CRC of r bits detects ALL burst errors of length ≤ r, all single-bit errors, all double-bit errors (if G(x) has ≥ 3 non-zero terms), any odd number of errors (if G(x) contains the factor x+1), and 1 − 2⁻ʳ of longer bursts.

Given data for this problem:

    - Message M(x) : 1 1 1 0 0 0 1 1  (8 bits)

    - Generator G(x) : Xs + 1 → binary 1 0 0 1 (4 bits, degree r = 3)

    - Number of appended zero bits : r = 3

    - Length of transmitted codeword : k + r = 8 + 3 = 11 bits

## Part (i) — Sender-Side Polynomial Long Division

Step 1: Multiply M(x) by xs (i.e. append 3 zeros to the right).

Augmented message = 1 1 1 0 0 0 1 1 0 0 0

Step 2: Divide 11100011000 by 1001 using modulo-2 (XOR) arithmetic. At each step, if the current leading bit is 1, XOR the divisor into the current position; if 0, shift and bring down the next bit.

1 1 1 0 0 0 1 1 0 0 0	← augmented dividend

÷ 1 0 0 1	← divisor G(x)

1 1 1 0 0 0 1 1 0 0 0

1 0 0 1

1 1 1 0 0

1 0 0 1

1 1 1 0 1

1 0 0 1

1 1 1 1 1

1 0 0 1

1 1 0 0 0

1 0 0 1

1 0 0 0

1 0 0 1

1 0 0	← Remainder R(x) = CRC / FCS

Step 3: The 3-bit CRC / FCS is R(x) = 100.

Step 4: Form the transmitted codeword T(x) = M(x)·xs + R(x):

Transmitted frame T(x) = 1 1 1 0 0 0 1 1 | 1 0 0

= 11100011100  (11 bits)

Verification: dividing 11100011100 by 1001 must produce remainder 000, confirming that the transmitted codeword is a multiple of the generator polynomial.

## Part (ii) — Effect of a Leftmost-Bit Error

Suppose channel noise inverts the leftmost bit of the transmitted frame. The received frame R′(x) becomes:

Transmitted : 1 1 1 0 0 0 1 1 1 0 0

Received	: 0 1 1 0 0 0 1 1 1 0 0	← bit 1 flipped

The receiver performs its own division of the entire received frame by the same generator 1001:

0 1 1 0 0 0 1 1 1 0 0

1 0 0 1

0 1 0 1 0 0 1 1 1 0 0

1 0 0 1

0 0 1 1 0 1 1 1 0 0

1 0 0 1

0 1 0 0 1 1 0 0

1 0 0 1

0 0 0 1 0 0

Final remainder S(x) = 1 0 1	(non-zero)

How the receiver knows an error has occurred:

- For an error-free reception, division of the received polynomial by G(x) yields remainder 000 by construction.

- Here the receiver obtains remainder 101 ≠ 000, which is called a non-zero syndrome.

- A non-zero syndrome mathematically guarantees that R′(x) is NOT a multiple of G(x), i.e. one or more bits were altered on the channel.

- The receiver therefore discards the frame; depending on the ARQ mechanism (Stop-and-Wait, Go-Back-N, or Selective Repeat), it either requests a retransmission (NAK / no ACK) or silently drops it and lets the sender’s timer expire.

Detection Strength of X³ + 1

Because G(x) = xs + 1 = (x + 1)(x² + x + 1), it contains the factor (x + 1), so it detects every error pattern with an odd number of bit errors. It also detects every burst error of length ≤ 3. It does, however, miss certain 2-bit error patterns spaced exactly by a multiple of 7 bits — which is why real networks use much longer generators such as CRC-16 or CRC-32.

# SECTION 2 — Throughput and Shannon Channel Capacity

## Bandwidth vs. Throughput vs. Goodput — A Quick Refresher

    - Bandwidth: The maximum theoretical data rate a link can carry, determined by the physical medium and encoding.

    - Throughput: The actual rate of successful data delivery, measured at the network layer, always less than bandwidth.

    - Goodput: Application-layer useful data, excluding retransmissions, headers, and control traffic.

Throughput < Bandwidth because of protocol overhead, collisions, MAC contention, propagation delays, buffer limits, congestion, and error retransmissions.

## Part (a) — Throughput Calculation

Given:

    - Link bandwidth = 10 Mbps

    - Frames per minute = 12,000

    - Average frame size = 10,000 bits Step 1: Total bits carried per minute.

Bits/minute = 12,000 frames × 10,000 bits = 120,000,000 bits

Step 2: Convert to bits per second.

Throughput = 120,000,000 / 60 = 2,000,000 bps = 2 Mbps

Step 3: Utilization = Throughput / Bandwidth = 2 / 10 = 0.20 = 20%.

Interpretation: Only one-fifth of the provisioned bandwidth is being used effectively. The remaining 80% is being lost to protocol overhead, idle time between frames, MAC arbitration, retransmissions, and possibly under-utilized upper-layer applications. To improve throughput one could use larger frames (reducing per-frame overhead), aggregate multiple packets, or eliminate half-duplex collisions with a switched full-duplex link.

Part (b) — Shannon Channel Capacity

Given:

Frequency band = 300 Hz to 3300 Hz → Bandwidth B = 3000 Hz

Signal-to-Noise Ratio (SNR, linear) = 3162

Shannon–Hartley theorem states the maximum error-free capacity of a noisy analog channel:

C = B × log₂(1 + SNR)

Step 1: Compute 1 + SNR.

1 + 3162 = 3163

Step 2: Compute log₂(3163).

log₂(3163) = log₁₀(3163) / log₁₀(2)

= 3.4999 / 0.30103

≈ 11.627

Step 3: Multiply by bandwidth.

C = 3000 × 11.627 ≈ 34,881 bps ≈ 34.88 kbps

Convert SNR to decibels for reference:

SNR (dB) = 10 log₁₀(3162) ≈ 35 dB

Interpretation: This 3000 Hz voice-grade channel with 35 dB SNR can never carry more than ~34.88 kbps of error-free information no matter what modulation is used. That is precisely why traditional dial-up modems capped out around V.34 (33.6 kbps). To exceed this limit, either the bandwidth (B) or the SNR must be increased — which is what broadband and fiber do.

Nyquist vs. Shannon — Comparison

| Aspect | Nyquist | Shannon |
| --- | --- | --- |
| Handles noise? | No (ideal noiseless channel) | Yes (includes SNR) |
| Formula | C = 2B log₂ L | C = B log₂(1 + SNR) |
| Determines | Max symbol rate | Max information rate |
| Given B & SNR here | Requires L | Directly ≈ 34.88 kbps |

SECTION 3 — Hamming (12, 8) Forward Error Correction

Scenario Recap

A multispecialty hospital uses wireless devices to continuously stream a patient’s heart rate and blood pressure to a central monitoring station. The wireless medium suffers occasional electromagnetic interference which flips a single bit. Because a critical-care doctor cannot wait for retransmission of a life-sign, the system uses Forward Error Correction (FEC) via a Hamming (12, 8) code: the receiver detects AND corrects the corrupted bit on the fly.

Code Parameters

Codeword length n = 12 bits

Data bits k = 8

Parity (redundant) bits r = n – k = 4

Hamming validity: 2ʳ ≥ k + r + 1 → 16 ≥ 13 ✓

Minimum Hamming distance dₘᵢₙ = 3 → corrects 1 bit, detects up to 2 bits

Code rate R = k / n = 8/12 ≈ 0.667

Bit-Position Layout

Parity bits occupy positions 1, 2, 4, 8 (all powers of two). Data bits fill the remaining positions.

Position : 1  2  3  4  5  6  7  8  9 10 11 12

Bit type : P1 P2 D1 P4 D2 D3 D4 P8 D5 D6 D7 D8

Parity Equations (Even Parity)

Each parity bit covers all positions whose binary representation has the corresponding bit set:

P1 covers positions 1,3,5,7,9,11 → P1 = D1 ⊕ D2 ⊕ D4 ⊕ D5 ⊕ D7 P2 covers positions 2,3,6,7,10,11 → P2 = D1 ⊕ D3 ⊕ D4 ⊕ D6 ⊕ D7 P4 covers positions 4,5,6,7,12  → P4 = D2 ⊕ D3 ⊕ D4 ⊕ D8

P8 covers positions 8,9,10,11,12  → P8 = D5 ⊕ D6 ⊕ D7 ⊕ D8

Worked Example — Encoding Data 10110010

Let D1..D8 = 1, 0, 1, 1, 0, 0, 1, 0.

P1 = 1 ⊕ 0 ⊕ 1 ⊕ 0 ⊕ 1 = 1

P2 = 1 ⊕ 1 ⊕ 1 ⊕ 0 ⊕ 1 = 0

P4 = 0 ⊕ 1 ⊕ 1 ⊕ 0 = 0

P8 = 0 ⊕ 0 ⊕ 1 ⊕ 0 = 1

Codeword (pos 1..12) = 1 0 1 0 0 1 1 1 0 0 1 0

Receiver Syndrome Decoding

The receiver recomputes C1, C2, C4, C8 using the same equations on the received bits. The binary number C8 C4 C2 C1 is the syndrome:

Syndrome = 0000 → no error, deliver data.

Syndrome ≠ 0000 → its decimal value is the position of the erroneous bit; flip that bit to correct.

Example — suppose bit 5 is flipped in transit. C1 = 1, C2 = 0, C4 = 1, C8 = 0 → syndrome = 0101₂ = 5. The receiver inverts bit 5 and the data is restored without any retransmission.

Why FEC (Not ARQ) for Hospital Monitoring

Zero retransmission latency — critical for real-time vital signs.

Constant-rate data stream even over lossy wireless.

Battery-powered patient device sends only forward — no reception window needed, saving power.

Cost: extra 50% overhead (4 parity bits per 8 data bits), but negligible for low-rate physiological data.

SECTION 4 — IPv4 Subnetting and IPv6 Addressing

Part (a) — Subnetting 172.16.10.0/24 into /27 Blocks

Original network: 172.16.10.0/24, subnetted with mask /27 (255.255.255.224).

Step 1 — Count Borrowed Bits

Borrowed bits = 27 – 24 = 3 Number of subnets = 2³ = 8

Step 2 — Hosts per Subnet

Host bits = 32 – 27 = 5

Total addresses/subnet = 2⁵ = 32 Usable hosts/subnet = 32 – 2 = 30

(subtract 1 for network address and 1 for broadcast address)

Step 3 — Full Subnet Table

| Subnet # | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- |
| 0 | 172.16.10.0 | 172.16.10.1 | 172.16.10.30 | 172.16.10.31 |
| 1 | 172.16.10.32 | 172.16.10.33 | 172.16.10.62 | 172.16.10.63 |
| 2 | 172.16.10.64 | 172.16.10.65 | 172.16.10.94 | 172.16.10.95 |
| 3 | 172.16.10.96 | 172.16.10.97 | 172.16.10.126 | 172.16.10.127 |
| 4 | 172.16.10.128 | 172.16.10.129 | 172.16.10.158 | 172.16.10.159 |
| 5 | 172.16.10.160 | 172.16.10.161 | 172.16.10.190 | 172.16.10.191 |
| 6 | 172.16.10.192 | 172.16.10.193 | 172.16.10.222 | 172.16.10.223 |
| 7 | 172.16.10.224 | 172.16.10.225 | 172.16.10.254 | 172.16.10.255 |

Step 4 — Locate 172.16.10.78

The last octet 78 falls between 64 and 95, so it belongs to Subnet #2.

Subnet: 172.16.10.64/27

Network address: 172.16.10.64

Broadcast address: 172.16.10.95

Usable host range: 172.16.10.65 – 172.16.10.94

Subnet mask: 255.255.255.224

Step 5 — Is 172.16.10.95 in the Same Subnet?

Yes. 172.16.10.95 lies exactly at the top of Subnet #2 (64–95). However, it is the broadcast address of that subnet and cannot be assigned to any host — assigning it would prevent normal unicast reception.

Part (b) — IPv6 Address 2001:0db8:0000:0000:0000:ff00:0042:8329

Compression Rules

In each 16-bit group, drop leading zeros (e.g. 0042 → 42, 0db8 → db8).

Replace exactly one longest run of consecutive all-zero groups with “::”.

“::” may appear at most once in an address to keep it unambiguous.

Compression

Full	: 2001:0db8:0000:0000:0000:ff00:0042:8329

Step 1 : strip leading zeros

2001: db8: 0 : 0 : 0 : ff00: 42 : 8329

Step 2 : collapse longest run of 0-groups (three of them) Compressed: 2001:db8::ff00:42:8329

Expansion Back to Full Form

Count the groups present (five: 2001, db8, ff00, 42, 8329).

Insert (8 – 5) = 3 zero groups where “::” appears.

Restore leading zeros so every group is 4 hex digits.

Expanded : 2001:0db8:0000:0000:0000:ff00:0042:8329

Network Prefix with /64

A /64 prefix keeps the first 64 bits (four 16-bit hextets):

Prefix : 2001:0db8:0000:0000::/64 (short form 2001:db8::/64) Interface ID (last 64 bits): 0000:ff00:0042:8329

Possible Host Addresses in a /64

Host bits	= 128 – 64 = 64

Total hosts	= 2⁶⁴ = 18,446,744,073,709,551,616

Because IPv6 has no broadcast concept (multicast and anycast replace it), all 2⁶⁴ addresses of a /64 are theoretically usable. The /64 boundary is also mandated by SLAAC (Stateless Address Autoconfiguration) which relies on the 64-bit interface identifier derived from EUI-64 or a privacy-generated random value.

Comparison — IPv4 vs. IPv6

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address size | 32 bits | 128 bits |
| Address space | ~4.3 × 10⁹ | ~3.4 × 10s⁸ |
| Notation | Dotted decimal | Colon-hex |
| Broadcast | Yes | No (multicast only) |
| Auto-config | DHCP | SLAAC + DHCPv6 |
| Header size | 20–60 bytes | Fixed 40 bytes |

# SECTION 5 — Case Study: Online Semester Examination

## Restating the Scenario

At 10:00 AM sharp, a university’s examination server begins transmitting encrypted question papers to 8,000 students simultaneously. The server sits on a 10 Gbps backbone; students connect via 100 Mbps fiber, 20 Mbps home Wi-Fi, or 5–10 Mbps mobile networks. Within minutes: incomplete downloads, repeated retransmissions, delayed ACKs, and portal freezes on slower connections. The server keeps pumping data at high speed even though many receiver devices cannot process packets that fast. Receiver buffers overflow, retransmissions multiply, and question-paper delivery is delayed.

## Primary Networking Problem — Flow-Control Failure

The core issue is a Flow-Control failure at the transport layer, accompanied by secondary congestion problems in the intermediate network. The sender’s outgoing data rate (10 Gbps aggregate) grossly exceeds the receivers’ ability to accept and process incoming packets (5 Mbps to 100 Mbps per student). When there is no explicit mechanism to inform the sender of the receiver’s current buffer availability — or when that mechanism is not respected — the receiver’s buffer fills up, incoming packets are silently dropped, and the sender’s retransmission timers add still more traffic to the already saturated pipe.

## Symptom-by-Symptom Analysis

| Symptom | Underlying Cause |
| --- | --- |
| Incomplete downloads | Packets dropped at overflowing receiver / router buffers |
| Repeated retransmissions | TCP timeout triggers RTO before ACK arrives |
| Delayed acknowledgments | Long queues raise RTT; ACKs stuck behind data on slow uplink |
| Temporary portal freeze | TCP head-of-line blocking waiting for missing segments |
| Receiver buffer overflow | Application drains buffer slower than TCP fills it |
| Server keeps transmitting | No or stale flow-control feedback (rwnd) |

## Why the Speed Mismatch Hurts Communication

    - Buffer overflow — A 5 Mbps mobile client receiving from a 10 Gbps server has an arrival-to-drain ratio of 2000:1. Its socket buffer fills in milliseconds; new packets are dropped.

    - Excess retransmissions — Every dropped packet triggers a retransmit that reoccupies scarce last-mile capacity, crowding out fresh data. Retransmissions can consume 30–80% of link bandwidth during severe overload.

    - ACK compression / delay — On congested uplinks, ACKs are queued behind data. The sender receives stale window advertisements and either sends too fast (persistent overload) or too slow (unnecessarily idle).

    - Head-of-line blocking — TCP delivers bytes in order, so one lost segment stalls all subsequent segments already in the buffer, making the portal appear to freeze.

    - Congestion collapse — If enough clients simultaneously retransmit, aggregate goodput can approach zero — the classic “congestion collapse” observed on the ARPANET in October 1986.

    - Fairness collapse — Faster fiber users still complete quickly, while mobile users experience worst-case exponential back-off, so the delay is not just longer, it is unequal.

## Recommended Solutions

### Transport-Layer Flow Control

  - TCP Sliding-Window Protocol: every ACK carries a 16-bit rwnd advertising free buffer space; the sender may never have more unacknowledged bytes on the wire than rwnd. Slow clients naturally shrink rwnd → the server slows down automatically.

  - Window Scaling Option (RFC 7323): required for high-BDP links so rwnd can exceed 64 KB.

### Network-Layer Congestion Control

  - Modern algorithms — CUBIC, BBR, PRR, TCP Reno — shrink cwnd on packet loss / RTT increase, matching aggregate demand to available capacity.

  - ECN (Explicit Congestion Notification) allows routers to mark packets instead of dropping them, giving earlier feedback.

### Application-Level Design

  - Staggered start: release question papers in waves (e.g. 500 students per 5 seconds) instead of a single 10:00 AM thundering herd.

  - CDN / edge servers: distribute the file so no single 10 Gbps pipe funnels 8,000 downloads.

  - Resumable HTTP Range requests: a dropped connection resumes from the last received byte instead of restarting.

  - Compression + lightweight text-only version for the weakest mobile links.

  - Adaptive rate limiting at the server (token bucket per client) to never exceed a client’s observed capacity.

### Operational Measures

  - Pre-cache the encrypted paper on each device before 10:00 AM, releasing only the decryption key at exam start.

  - Use QUIC / HTTP/3 over UDP which avoids TCP’s head-of-line blocking entirely.

  - Enforce QoS at the edge to prioritize small ACKs over large data on the uplink.

## Conclusion

The examination portal failure is not a bandwidth problem — the server has 10 Gbps to spare. It is a Flow-Control problem: the sender is faster than the receivers, and there is no effective feedback loop throttling it to the slowest link. Implementing proper TCP flow control (sliding window with honoured rwnd), congestion-aware algorithms (CUBIC / BBR), and application-level staggering with a CDN would eliminate the buffer overflows, retransmissions and portal freezes described in the scenario.

# RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient ) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understandin g | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justification | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structure d code with proper comments and documentatio n | Code is mostly clear with minor documentat ion gaps | Code lacks structure and proper documentatio n | Poorly written code with no documentatio n |