# SIMATS ENGINEERING

## ASSESSMENT TOOL 1

### COURSE CODE: CSA0713	COURSE NAME: COMPUTER NETWORKS

### COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

## ANALYTICAL PROBLEM SOLVING

#### Code of Conduct:

I S.HARIPRIYA(192412448) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

s.haripriyaa

#### Signature of the student:

Annexure A

## ANALYTICAL PROBLEM SOLVING

- Suppose we want to transmit the message 11100011 and protect it from errors using the CRC polynomial X3+1.

i.) Use polynomial long division to determine the message that should be transmitted.

ii) Suppose the leftmost bit of the message is inverted due to noise on the transmission link. What is the result of the receiver's CRC calculation? How does the receiver know that an error has occurred?

- a) A network with bandwidth of 10 Mbps can pass only an average of 12,000 frames per minute with each frame carrying an average of 10,000 bits. What is the throughput of this network?

b) A network has a bandwidth of 3000 Hz (300 to 3300 Hz) assigned for data communications. The signal- to-noise ratio is 3162. Find the channel capacity

- A multispecialty hospital uses wireless patient-monitoring devices to continuously transmit patients' heart rate and blood pressure readings to a central monitoring station. Since the data is transmitted over a wireless channel, electromagnetic interference occasionally changes one bit during transmission. To ensure that doctors receive accurate patient information without waiting for retransmission, the hospital implements the Hamming (12,8) error-correcting code.

- a) A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a

/27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

b) A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full

notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

- A university is conducting an online semester examination for 8,000 students simultaneously. At exactly 10:00 AM, the examination server begins transmitting encrypted question papers to all students. The university server is connected through a 10 Gbps high-speed network, while students access the examination portal using different internet connections such as fiber broadband (100 Mbps), home Wi-Fi (20 Mbps), and mobile networks (5–10 Mbps). During the first few minutes of the examination, many students with slower internet connections report incomplete downloads, repeated retransmissions, delayed acknowledgments, and temporary freezing of the examination portal. Network administrators observe that the server continues transmitting data at a high rate even though several student devices are unable to receive and process packets at the same speed. This results in receiver buffer overflow, unnecessary retransmissions, increased network traffic, and delays in delivering the question papers. Analyze the above scenario and identify the primary networking problem. Explain how the mismatch between the sender's transmission speed and the receiver's processing capability affects communication.

# Annexure A — Analytical Problem Solving

Detailed, Elaborated Solutions — Version 1

This report presents exhaustive, step-by-step solutions to five analytical problems in computer networks:

(1) error detection using the CRC generator polynomial Xs + 1; (2) network throughput and Shannon channel capacity; (3) forward error correction using the Hamming (12, 8) code for a hospital wireless monitoring system; (4) IPv4 subnetting of 172.16.10.0/24 with a /27 mask and IPv6 addressing rules for 2001:0db8:0000:0000:0000:ff00:0042:8329; and (5) a case study on flow control during an online semester examination for 8,000 students. Each question is answered with full theory, formulas, arithmetic, worked examples and interpretation.

Table of Contents:

- Section 1 — Cyclic Redundancy Check (CRC) using Xs + 1

- Section 2 — Throughput and Shannon Channel Capacity

- Section 3 — Hamming (12, 8) Forward Error Correction

- Section 4 — IPv4 Subnetting and IPv6 Addressing

- Section 5 — Case Study: Online Semester Examination

SECTION 1 — Cyclic Redundancy Check (CRC) using X³ + 1

## Introduction and Theory

Cyclic Redundancy Check (CRC) is a very powerful and easy-to-implement error-detection technique used in nearly every modern data-link layer protocol — Ethernet (CRC-32), Wi-Fi, Bluetooth, HDLC, USB, and even in ZIP and PNG file formats. The core idea is to treat any block of k message bits as the coefficients of a polynomial M(x) of degree k–1, and then divide it (modulo-2, i.e. bitwise XOR without carries) by an agreed-upon generator polynomial G(x) of degree r. The r-bit remainder R(x) produced by this division is appended to the message and transmitted as the Frame Check Sequence (FCS).

Why CRC is preferred over a simple checksum or parity bit:

    - A single parity bit detects only an odd number of bit errors.

    - A checksum can miss reordered or paired errors.

    - A well-chosen CRC of r bits detects ALL burst errors of length ≤ r, all single-bit errors, all double-bit errors (if G(x) has ≥ 3 non-zero terms), any odd number of errors (if G(x) contains the factor x+1), and 1 − 2⁻ʳ of longer bursts.

Given data for this problem:

    - Message M(x) : 1 1 1 0 0 0 1 1  (8 bits)

    - Generator G(x) : Xs + 1 → binary 1 0 0 1 (4 bits, degree r = 3)

    - Number of appended zero bits : r = 3

    - Length of transmitted codeword : k + r = 8 + 3 = 11 bits

## Part (i) — Sender-Side Polynomial Long Division

Step 1: Multiply M(x) by xs (i.e. append 3 zeros to the right).

Augmented message = 1 1 1 0 0 0 1 1 0 0 0

Step 2: Divide 11100011000 by 1001 using modulo-2 (XOR) arithmetic. At each step, if the current leading bit is 1, XOR the divisor into the current position; if 0, shift and bring down the next bit.

1 1 1 0 0 0 1 1 0 0 0	← augmented dividend

÷ 1 0 0 1	← divisor G(x)

1 1 1 0 0 0 1 1 0 0 0

1 0 0 1

1 1 1 0 0

1 0 0 1

1 1 1 0 1

1 0 0 1

1 1 1 1 1

1 0 0 1

1 1 0 0 0

1 0 0 1

1 0 0 0

1 0 0 1

1 0 0	← Remainder R(x) = CRC / FCS

Step 3: The 3-bit CRC / FCS is R(x) = 100.

Step 4: Form the transmitted codeword T(x) = M(x)·xs + R(x):

Transmitted frame T(x) = 1 1 1 0 0 0 1 1 | 1 0 0

= 11100011100  (11 bits)

Verification: dividing 11100011100 by 1001 must produce remainder 000, confirming that the transmitted codeword is a multiple of the generator polynomial.

## Part (ii) — Effect of a Leftmost-Bit Error

Suppose channel noise inverts the leftmost bit of the transmitted frame. The received frame R′(x) becomes:

Transmitted : 1 1 1 0 0 0 1 1 1 0 0

Received	: 0 1 1 0 0 0 1 1 1 0 0	← bit 1 flipped

The receiver performs its own division of the entire received frame by the same generator 1001:

0 1 1 0 0 0 1 1 1 0 0

1 0 0 1

0 1 0 1 0 0 1 1 1 0 0

1 0 0 1

0 0 1 1 0 1 1 1 0 0

1 0 0 1

0 1 0 0 1 1 0 0

1 0 0 1

0 0 0 1 0 0

Final remainder S(x) = 1 0 1	(non-zero)

How the receiver knows an error has occurred:

- For an error-free reception, division of the received polynomial by G(x) yields remainder 000 by construction.

- Here the receiver obtains remainder 101 ≠ 000, which is called a non-zero syndrome.

- A non-zero syndrome mathematically guarantees that R′(x) is NOT a multiple of G(x), i.e. one or more bits were altered on the channel.

- The receiver therefore discards the frame; depending on the ARQ mechanism (Stop-and-Wait, Go-Back-N, or Selective Repeat), it either requests a retransmission (NAK / no ACK) or silently drops it and lets the sender’s timer expire.

Detection Strength of X³ + 1

Because G(x) = xs + 1 = (x + 1)(x² + x + 1), it contains the factor (x + 1), so it detects every error pattern with an odd number of bit errors. It also detects every burst error of length ≤ 3. It does, however, miss certain 2-bit error patterns spaced exactly by a multiple of 7 bits — which is why real networks use much longer generators such as CRC-16 or CRC-32.

# SECTION 2 — Throughput and Shannon Channel Capacity

## Bandwidth vs. Throughput vs. Goodput — A Quick Refresher

    - Bandwidth: The maximum theoretical data rate a link can carry, determined by the physical medium and encoding.

    - Throughput: The actual rate of successful data delivery, measured at the network layer, always less than bandwidth.

    - Goodput: Application-layer useful data, excluding retransmissions, headers, and control traffic.

Throughput < Bandwidth because of protocol overhead, collisions, MAC contention, propagation delays, buffer limits, congestion, and error retransmissions.

## Part (a) — Throughput Calculation

Given:

    - Link bandwidth = 10 Mbps

    - Frames per minute = 12,000

    - Average frame size = 10,000 bits Step 1: Total bits carried per minute.

Bits/minute = 12,000 frames × 10,000 bits = 120,000,000 bits

Step 2: Convert to bits per second.

Throughput = 120,000,000 / 60 = 2,000,000 bps = 2 Mbps

Step 3: Utilization = Throughput / Bandwidth = 2 / 10 = 0.20 = 20%.

Interpretation: Only one-fifth of the provisioned bandwidth is being used effectively. The remaining 80% is being lost to protocol overhead, idle time between frames, MAC arbitration, retransmissions, and possibly under-utilized upper-layer applications. To improve throughput one could use larger frames (reducing per-frame overhead), aggregate multiple packets, or eliminate half-duplex collisions with a switched full-duplex link.

## Part (b) — Shannon Channel Capacity

Given:

    - Frequency band = 300 Hz to 3300 Hz → Bandwidth B = 3000 Hz

    - Signal-to-Noise Ratio (SNR, linear) = 3162

Shannon–Hartley theorem states the maximum error-free capacity of a noisy analog channel:

C = B × log₂(1 + SNR)

Step 1: Compute 1 + SNR.

1 + 3162 = 3163

Step 2: Compute log₂(3163).

log₂(3163) = log₁₀(3163) / log₁₀(2)

= 3.4999 / 0.30103

≈ 11.627

Step 3: Multiply by bandwidth.

C = 3000 × 11.627 ≈ 34,881 bps ≈ 34.88 kbps

Convert SNR to decibels for reference:

SNR (dB) = 10 log₁₀(3162) ≈ 35 dB

Interpretation: This 3000 Hz voice-grade channel with 35 dB SNR can never carry more than ~34.88 kbps of error-free information no matter what modulation is used. That is precisely why traditional dial-up modems capped out around V.34 (33.6 kbps). To exceed this limit, either the bandwidth (B) or the SNR must be increased — which is what broadband and fiber do.

## Nyquist vs. Shannon — Comparison

| Aspect | Nyquist | Shannon |
| --- | --- | --- |
| Handles noise? | No (ideal noiseless channel) | Yes (includes SNR) |
| Formula | C = 2B log₂ L | C = B log₂(1 + SNR) |
| Determines | Max symbol rate | Max information rate |
| Given B & SNR here | Requires L | Directly ≈ 34.88 kbps |

# SECTION 3 — Hamming (12, 8) Forward Error Correction

## Scenario Recap

A multispecialty hospital uses wireless devices to continuously stream a patient’s heart rate and blood pressure to a central monitoring station. The wireless medium suffers occasional electromagnetic interference which flips a single bit. Because a critical-care doctor cannot wait for retransmission of a life-sign, the system uses Forward Error Correction (FEC) via a Hamming (12, 8) code: the receiver detects AND corrects the corrupted bit on the fly.

## Code Parameters

    - Codeword length n = 12 bits

    - Data bits k = 8

    - Parity (redundant) bits r = n – k = 4

    - Hamming validity: 2ʳ ≥ k + r + 1 → 16 ≥ 13 ✓

    - Minimum Hamming distance dₘᵢₙ = 3 → corrects 1 bit, detects up to 2 bits

    - Code rate R = k / n = 8/12 ≈ 0.667

## Bit-Position Layout

Parity bits occupy positions 1, 2, 4, 8 (all powers of two). Data bits fill the remaining positions.

Position : 1  2  3  4  5  6  7  8  9 10 11 12

Bit type : P1 P2 D1 P4 D2 D3 D4 P8 D5 D6 D7 D8

## Parity Equations (Even Parity)

Each parity bit covers all positions whose binary representation has the corresponding bit set:

P1 covers positions 1,3,5,7,9,11 → P1 = D1 ⊕ D2 ⊕ D4 ⊕ D5 ⊕ D7 P2 covers positions 2,3,6,7,10,11 → P2 = D1 ⊕ D3 ⊕ D4 ⊕ D6 ⊕ D7 P4 covers positions 4,5,6,7,12  → P4 = D2 ⊕ D3 ⊕ D4 ⊕ D8

P8 covers positions 8,9,10,11,12  → P8 = D5 ⊕ D6 ⊕ D7 ⊕ D8

## Worked Example — Encoding Data 10110010

Let D1..D8 = 1, 0, 1, 1, 0, 0, 1, 0.

P1 = 1 ⊕ 0 ⊕ 1 ⊕ 0 ⊕ 1 = 1

P2 = 1 ⊕ 1 ⊕ 1 ⊕ 0 ⊕ 1 = 0

P4 = 0 ⊕ 1 ⊕ 1 ⊕ 0 = 0

P8 = 0 ⊕ 0 ⊕ 1 ⊕ 0 = 1

Codeword (pos 1..12) = 1 0 1 0 0 1 1 1 0 0 1 0

## Receiver Syndrome Decoding

The receiver recomputes C1, C2, C4, C8 using the same equations on the received bits. The binary number C8 C4 C2 C1 is the syndrome:

    - Syndrome = 0000 → no error, deliver data.

    - Syndrome ≠ 0000 → its decimal value is the position of the erroneous bit; flip that bit to correct.

Example — suppose bit 5 is flipped in transit. C1 = 1, C2 = 0, C4 = 1, C8 = 0 → syndrome = 0101₂ = 5. The receiver inverts bit 5 and the data is restored without any retransmission.

## Why FEC (Not ARQ) for Hospital Monitoring

    - Zero retransmission latency — critical for real-time vital signs.

    - Constant-rate data stream even over lossy wireless.

    - Battery-powered patient device sends only forward — no reception window needed, saving power.

    - Cost: extra 50% overhead (4 parity bits per 8 data bits), but negligible for low-rate physiological data.

# SECTION 4 — IPv4 Subnetting and IPv6 Addressing

## Part (a) — Subnetting 172.16.10.0/24 into /27 Blocks

Original network: 172.16.10.0/24, subnetted with mask /27 (255.255.255.224).

### Step 1 — Count Borrowed Bits

Borrowed bits = 27 – 24 = 3 Number of subnets = 2³ = 8

### Step 2 — Hosts per Subnet

Host bits = 32 – 27 = 5

Total addresses/subnet = 2⁵ = 32 Usable hosts/subnet = 32 – 2 = 30

(subtract 1 for network address and 1 for broadcast address)

### Step 3 — Full Subnet Table

| Subnet # | Network Address | First Host | Last Host | Broadcast |
| --- | --- | --- | --- | --- |
| 0 | 172.16.10.0 | 172.16.10.1 | 172.16.10.30 | 172.16.10.31 |
| 1 | 172.16.10.32 | 172.16.10.33 | 172.16.10.62 | 172.16.10.63 |
| 2 | 172.16.10.64 | 172.16.10.65 | 172.16.10.94 | 172.16.10.95 |
| 3 | 172.16.10.96 | 172.16.10.97 | 172.16.10.126 | 172.16.10.127 |
| 4 | 172.16.10.128 | 172.16.10.129 | 172.16.10.158 | 172.16.10.159 |
| 5 | 172.16.10.160 | 172.16.10.161 | 172.16.10.190 | 172.16.10.191 |
| 6 | 172.16.10.192 | 172.16.10.193 | 172.16.10.222 | 172.16.10.223 |
| 7 | 172.16.10.224 | 172.16.10.225 | 172.16.10.254 | 172.16.10.255 |

### Step 4 — Locate 172.16.10.78

The last octet 78 falls between 64 and 95, so it belongs to Subnet #2.

- Subnet: 172.16.10.64/27

- Network address: 172.16.10.64

- Broadcast address: 172.16.10.95

- Usable host range: 172.16.10.65 – 172.16.10.94

- Subnet mask: 255.255.255.224

### Step 5 — Is 172.16.10.95 in the Same Subnet?

Yes. 172.16.10.95 lies exactly at the top of Subnet #2 (64–95). However, it is the broadcast address of that subnet and cannot be assigned to any host — assigning it would prevent normal unicast reception.

## Part (b) — IPv6 Address 2001:0db8:0000:0000:0000:ff00:0042:8329

### Compression Rules

    - In each 16-bit group, drop leading zeros (e.g. 0042 → 42, 0db8 → db8).

    - Replace exactly one longest run of consecutive all-zero groups with “::”.

    - “::” may appear at most once in an address to keep it unambiguous.

### Compression

Full	: 2001:0db8:0000:0000:0000:ff00:0042:8329

Step 1 : strip leading zeros

2001: db8: 0 : 0 : 0 : ff00: 42 : 8329

Step 2 : collapse longest run of 0-groups (three of them) Compressed: 2001:db8::ff00:42:8329

### Expansion Back to Full Form

- Count the groups present (five: 2001, db8, ff00, 42, 8329).

- Insert (8 – 5) = 3 zero groups where “::” appears.

- Restore leading zeros so every group is 4 hex digits.

Expanded : 2001:0db8:0000:0000:0000:ff00:0042:8329

### Network Prefix with /64

A /64 prefix keeps the first 64 bits (four 16-bit hextets):

Prefix : 2001:0db8:0000:0000::/64 (short form 2001:db8::/64) Interface ID (last 64 bits): 0000:ff00:0042:8329

### Possible Host Addresses in a /64

Host bits	= 128 – 64 = 64

Total hosts	= 2⁶⁴ = 18,446,744,073,709,551,616

Because IPv6 has no broadcast concept (multicast and anycast replace it), all 2⁶⁴ addresses of a /64 are theoretically usable. The /64 boundary is also mandated by SLAAC (Stateless Address Autoconfiguration) which relies on the 64-bit interface identifier derived from EUI-64 or a privacy-generated random value.

## Comparison — IPv4 vs. IPv6

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address size | 32 bits | 128 bits |
| Address space | ~4.3 × 10⁹ | ~3.4 × 10s⁸ |
| Notation | Dotted decimal | Colon-hex |
| Broadcast | Yes | No (multicast only) |
| Auto-config | DHCP | SLAAC + DHCPv6 |
| Header size | 20–60 bytes | Fixed 40 bytes |

# SECTION 5 — Case Study: Online Semester Examination

## Restating the Scenario

At 10:00 AM sharp, a university’s examination server begins transmitting encrypted question papers to 8,000 students simultaneously. The server sits on a 10 Gbps backbone; students connect via 100 Mbps fiber, 20 Mbps home Wi-Fi, or 5–10 Mbps mobile networks. Within minutes: incomplete downloads, repeated retransmissions, delayed ACKs, and portal freezes on slower connections. The server keeps pumping data at high speed even though many receiver devices cannot process packets that fast. Receiver buffers overflow, retransmissions multiply, and question-paper delivery is delayed.

## Primary Networking Problem — Flow-Control Failure

The core issue is a Flow-Control failure at the transport layer, accompanied by secondary congestion problems in the intermediate network. The sender’s outgoing data rate (10 Gbps aggregate) grossly exceeds the receivers’ ability to accept and process incoming packets (5 Mbps to 100 Mbps per student). When there is no explicit mechanism to inform the sender of the receiver’s current buffer availability — or when that mechanism is not respected — the receiver’s buffer fills up, incoming packets are silently dropped, and the sender’s retransmission timers add still more traffic to the already saturated pipe.

## Symptom-by-Symptom Analysis

| Symptom | Underlying Cause |
| --- | --- |
| Incomplete downloads | Packets dropped at overflowing receiver / router buffers |
| Repeated retransmissions | TCP timeout triggers RTO before ACK arrives |
| Delayed acknowledgments | Long queues raise RTT; ACKs stuck behind data on slow uplink |
| Temporary portal freeze | TCP head-of-line blocking waiting for missing segments |
| Receiver buffer overflow | Application drains buffer slower than TCP fills it |
| Server keeps transmitting | No or stale flow-control feedback (rwnd) |

## Why the Speed Mismatch Hurts Communication

    - Buffer overflow — A 5 Mbps mobile client receiving from a 10 Gbps server has an arrival-to-drain ratio of 2000:1. Its socket buffer fills in milliseconds; new packets are dropped.

    - Excess retransmissions — Every dropped packet triggers a retransmit that reoccupies scarce last-mile capacity, crowding out fresh data. Retransmissions can consume 30–80% of link bandwidth during severe overload.

    - ACK compression / delay — On congested uplinks, ACKs are queued behind data. The sender receives stale window advertisements and either sends too fast (persistent overload) or too slow (unnecessarily idle).

    - Head-of-line blocking — TCP delivers bytes in order, so one lost segment stalls all subsequent segments already in the buffer, making the portal appear to freeze.

    - Congestion collapse — If enough clients simultaneously retransmit, aggregate goodput can approach zero — the classic “congestion collapse” observed on the ARPANET in October 1986.

    - Fairness collapse — Faster fiber users still complete quickly, while mobile users experience worst-case exponential back-off, so the delay is not just longer, it is unequal.

## Recommended Solutions

### Transport-Layer Flow Control

  - TCP Sliding-Window Protocol: every ACK carries a 16-bit rwnd advertising free buffer space; the sender may never have more unacknowledged bytes on the wire than rwnd. Slow clients naturally shrink rwnd → the server slows down automatically.

  - Window Scaling Option (RFC 7323): required for high-BDP links so rwnd can exceed 64 KB.

### Network-Layer Congestion Control

  - Modern algorithms — CUBIC, BBR, PRR, TCP Reno — shrink cwnd on packet loss / RTT increase, matching aggregate demand to available capacity.

  - ECN (Explicit Congestion Notification) allows routers to mark packets instead of dropping them, giving earlier feedback.

### Application-Level Design

  - Staggered start: release question papers in waves (e.g. 500 students per 5 seconds) instead of a single 10:00 AM thundering herd.

  - CDN / edge servers: distribute the file so no single 10 Gbps pipe funnels 8,000 downloads.

  - Resumable HTTP Range requests: a dropped connection resumes from the last received byte instead of restarting.

  - Compression + lightweight text-only version for the weakest mobile links.

  - Adaptive rate limiting at the server (token bucket per client) to never exceed a client’s observed capacity.

### Operational Measures

  - Pre-cache the encrypted paper on each device before 10:00 AM, releasing only the decryption key at exam start.

  - Use QUIC / HTTP/3 over UDP which avoids TCP’s head-of-line blocking entirely.

  - Enforce QoS at the edge to prioritize small ACKs over large data on the uplink.

## Conclusion

The examination portal failure is not a bandwidth problem — the server has 10 Gbps to spare. It is a Flow-Control problem: the sender is faster than the receivers, and there is no effective feedback loop throttling it to the slowest link. Implementing proper TCP flow control (sliding window with honoured rwnd), congestion-aware algorithms (CUBIC / BBR), and application-level staggering with a CDN would eliminate the buffer overflows, retransmissions and portal freezes described in the scenario.