SIMATS ENGINEERING ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA0713

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Dr.V.Parthipan

Annexure A

Debugging & Optimisation Task

# Code of Conduct:

I s.haripriya(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

# Signature of the student:

# priyaaa

Annexure A

Debugging & Optimisation Task

- A rapidly growing IT company has shifted its operations to a newly constructed six-storey smart office building. More than 1,200 employees use wireless laptops, smartphones, IP phones, printers, and IoT devices connected through the company's Wireless Local Area Network (WLAN). The office relies heavily on cloud computing, online meetings, video conferencing, and file sharing, making uninterrupted wireless connectivity essential for daily operations. In addition, the company has a team of field engineers and sales executives who travel across different cities and use 4G/5G Wireless Wide Area Network (WWAN) services to access the company's servers, customer databases, and business applications from remote locations.

During the last few weeks, employees have reported several connectivity issues. Users experience slow internet speeds during peak working hours, frequent Wi-Fi disconnections while moving between floors, and weak wireless signals in conference rooms. Video conferencing applications often freeze or disconnect, affecting communication with international clients. Network administrators have also observed that some wireless access points are overloaded while others remain underutilized, leading to uneven traffic distribution. Furthermore, field engineers working in rural and highway areas frequently lose WWAN connectivity, delaying the upload of maintenance reports and customer information. Security audits reveal that a few wireless devices are still connected using outdated security protocols, exposing sensitive company information to potential cyber threats. The management has requested the network engineering team to investigate these issues, identify the root causes, and optimize both the WLAN and WWAN infrastructure to ensure high performance, secure communication, and seamless connectivity for all users.

- A multinational financial organization operates its headquarters, regional offices, and multiple data centers using an Asynchronous Transfer Mode (ATM) network to support mission-critical services such as online banking, video conferencing, Voice over IP (VoIP), and real-time financial transactions. The ATM network uses fixed-size 53-byte cells and establishes Virtual Circuits (VCs) to provide reliable and predictable Quality of Service (QoS). Recently, the organization has experienced several performance issues during peak business hours. Customers report delays in online transactions, voice calls suffer from jitter and occasional packet loss, and video conferencing sessions experience reduced quality. Network monitoring indicates that several ATM switches are operating at high utilization, some Virtual Circuits are congested, and available bandwidth is not being efficiently allocated among different traffic classes. Engineers also discover that certain applications are using inappropriate ATM service categories, leading to increased delay and inefficient resource utilization. The IT department has assigned you as the network engineer to investigate the ATM network, identify the faults, debug the performance issues, and recommend optimization strategies to restore efficient, reliable, and high-quality communication across the enterprise.

- A multinational e-commerce company has interconnected its headquarters, regional offices, warehouses, and cloud data centers using a Multiprotocol Label Switching (MPLS) network to ensure fast, secure, and reliable communication. The MPLS infrastructure supports critical business applications, including online order processing, enterprise

resource planning (ERP), video conferencing, Voice over IP (VoIP), cloud-based inventory management, and real-time customer support. Recently, the organization has experienced frequent performance issues during peak business hours. Employees report slow access to cloud applications, delayed order processing, poor-quality voice calls, and interruptions during video conferences. Network monitoring reveals that some Label Switched Paths (LSPs) are heavily congested, while other paths remain underutilized. The IT team also discovers that several routers have incorrect label mappings, causing inefficient routing and increased latency. In addition, traffic engineering has not been properly configured, resulting in uneven bandwidth utilization across the MPLS backbone. Since these issues are affecting customer satisfaction and business operations, the network administrator has asked you to investigate the MPLS network, identify the faults, debug the performance problems, and recommend suitable optimization techniques to improve the overall efficiency and reliability of the network.

- A multinational logistics company operates an automated warehouse where barcode scanners, RFID readers, autonomous robots, and inventory management systems continuously exchange data over a computer network. Every day, thousands of shipment records, customer details, and inventory updates are transmitted between warehouse terminals and the central server. Recently, the company has observed that some shipment records are arriving with incorrect information, barcode scans occasionally fail, and inventory quantities in the database do not always match the physical stock. During peak hours, network monitoring reports an increase in transmission errors caused by electrical interference, signal attenuation, and network congestion. Although some corrupted frames are detected and retransmitted, others remain undetected, resulting in incorrect database entries and delayed order processing. The IT department suspects that the current error detection and correction mechanisms are not configured efficiently. As the network engineer, you are assigned to investigate the transmission errors, identify the weaknesses in the existing error control techniques, and recommend suitable debugging and optimization strategies to improve the reliability and accuracy of data communication.

- A leading online education platform conducts live classes and online examinations for more than 15,000 students across different locations. During examinations, the central server simultaneously sends question papers, images, videos, and answer submission acknowledgements to all students over the network. The server is equipped with a high-speed 10 Gbps connection, whereas many students access the system using slower broadband or mobile internet connections. During peak hours, students experience incomplete downloads, delayed page loading, duplicate packets, and repeated retransmissions. Some students receive the next set of data before their devices finish processing the previous one, causing receiver buffer overflow and application crashes. Network administrators also observe an excessive number of retransmissions, increased latency, reduced throughput, and poor bandwidth utilization. The current flow control mechanism is not effectively regulating the transmission rate between the sender and receivers. As the network engineer, you have been assigned to investigate the communication problems, identify the causes of inefficient flow control, debug the existing system, and recommend suitable optimization techniques to ensure reliable and efficient data transmission during high-traffic periods.

Question 1 — Smart Office Building: WLAN and WWAN Optimization

A rapidly growing IT company with 1,200+ employees faces slow Wi-Fi during peak hours, disconnections between floors, weak signals in conference rooms, uneven AP load distribution, WWAN dropouts for field engineers, and outdated wireless security protocols. Investigate the root causes and recommend optimization strategies for both WLAN and WWAN infrastructure.

Answer

1. Problem Identification

A careful reading of the case reveals several distinct but interrelated fault categories that must be separated before any fix is attempted, because a single symptom such as “slow internet” can arise from RF issues, capacity issues, or protocol issues independently:

- Capacity/Throughput bottleneck: Over 1,200 devices across six floors are competing for a limited number of access points (APs) and shared channel bandwidth, causing congestion during peak business hours (typically 10 AM–1 PM and 3–6 PM).

- Roaming failure: Frequent disconnections while moving between floors indicate poor AP hand-off configuration — likely due to inconsistent SSID/channel planning or missing 802.11k/v/r fast-roaming support.

- RF coverage gaps: Weak signals in conference rooms point to physical attenuation from concrete/glass partitions, AP placement far from high-density zones, or insufficient transmit power tuning.

- Load imbalance: Some APs overloaded while others are idle suggests static, non-adaptive channel and power assignment rather than dynamic RF management (e.g., no use of Radio Resource Management/Auto Channel Selection).

- WWAN reliability: Field engineers on 4G/5G lose connectivity in rural/highway zones — a coverage and hand-off issue between cellular towers, compounded by lack of failover/redundant connectivity (e.g., no SD-WAN or multi-carrier bonding).

- Security vulnerability: Devices still using WEP/WPA/WPA2-Personal or open authentication expose the network to eavesdropping, deauthentication attacks, and credential theft.

2. Root Cause Analysis

Using a structured fault-isolation approach (physical layer → data link layer → network layer → application layer), the causes can be mapped as follows: at the physical layer, signal attenuation through walls and floor slabs and AP under-provisioning create dead zones; at the data link layer, channel overlap (co-channel interference) between adjacent APs operating on overlapping 2.4 GHz channels causes collisions and retransmissions; at the network layer, all devices may be placed on a single flat subnet/VLAN, so broadcast traffic from IoT devices, printers, and IP phones floods the same broadcast domain and starves bandwidth for video conferencing; and at the application layer, video conferencing (a real-time, delay-sensitive application) has no Quality of Service (QoS) prioritization, so it competes equally with best-effort file downloads.

For WWAN, the root cause of dropouts in rural and highway areas is a coverage-hole problem inherent to sparse cellular tower density, worsened by the absence of any redundancy mechanism such as dual-SIM failover or store-and-forward offline data caching on the field devices.

3. Debugging Methodology

The recommended debugging approach follows a systematic, bottom-up methodology:

- Conduct a wireless site survey (predictive and on-site) using tools such as Ekahau or NetSpot to map signal strength (RSSI), channel utilization, and interference across all six floors, identifying exact dead zones and overlapping-channel APs.

- Analyze WLAN controller logs to identify client roaming failures, authentication timeouts, and AP association/de-association patterns during peak hours.

- Use packet capture (Wireshark) on a sample video-conferencing session to confirm jitter, latency, and packet loss, and correlate the timestamps with AP load statistics.

- Audit the WWAN devices' carrier signal logs to identify specific geographic dead zones and correlate them with cell tower coverage maps.

- Run a security audit/penetration scan to enumerate every device still authenticating with outdated protocols (WEP/WPA/WPA2-PSK) and outdated firmware.

4. Optimization Strategies

Based on the debugging findings, the following optimization measures are recommended:

- Deploy additional APs using a capacity-based design (not just coverage-based), targeting no more than 25–30 concurrent clients per AP, with dedicated APs in high-density conference rooms.

- Enable 5 GHz and 6 GHz (Wi-Fi 6E) bands preferentially for high-bandwidth devices to reduce contention on the crowded 2.4 GHz band, and configure non-overlapping channels (1, 6, 11 on 2.4 GHz).

- Implement centralized WLAN controllers with Radio Resource Management (RRM) for dynamic channel and power allocation, automatically rebalancing load across APs.

- Enable 802.11k/v/r for fast, seamless roaming between floors so that VoIP and video calls do not drop during hand-off.

- Segment the network using VLANs (e.g., separate VLANs for IoT, guest, VoIP, and corporate data) combined with an SSID-to-VLAN mapping, reducing broadcast domain size.

- Apply QoS (WMM / DSCP marking) to prioritize VoIP and video-conferencing traffic over bulk data transfers.

- For WWAN, adopt SD-WAN with multi-carrier bonding or dual-SIM failover so field engineers automatically switch to the strongest available carrier, and enable offline caching/queued synchronization for maintenance reports in no-signal zones.

- Enforce WPA3-Enterprise (802.1X with RADIUS authentication) across all APs and disable legacy protocols entirely; deploy a Network Access Control (NAC) solution to quarantine non-compliant devices.

5. Analysis and Justification

The proposed solution directly targets each identified root cause rather than treating symptoms: capacity-based AP redesign resolves congestion, RRM and non-overlapping channel planning eliminate co-channel interference, VLAN segmentation and QoS protect latency-sensitive traffic, and WPA3-Enterprise closes the security gap without impacting usability. For WWAN, SD-WAN failover directly addresses the intermittent coverage problem by providing automatic redundancy rather than relying on a single carrier. Collectively, these measures improve throughput, reduce jitter for video calls, ensure secure authentication, and provide resilient connectivity for both fixed WLAN users and mobile WWAN field staff, meeting the company's requirement for uninterrupted, secure, high-performance connectivity.

Question 2 — ATM Network Performance Debugging and Optimization

A multinational financial organization runs a 53-byte cell, Virtual-Circuit-based ATM network for online banking, VoIP, and video conferencing. Customers face transaction delays, voice jitter/packet loss, and poor video quality. Some switches show high utilization, VCs are congested, and applications use inappropriate ATM service categories. Investigate, debug, and recommend optimization strategies.

Answer

1. Problem Identification

- High switch utilization during peak hours indicates the ATM switching fabric is operating near or above its engineered capacity, leading to cell queuing delay.

- Congested Virtual Circuits (VCs) suggest that Permanent Virtual Circuits (PVCs) or Switched Virtual Circuits (SVCs) for critical applications have insufficient bandwidth reservation (Peak Cell Rate/Sustainable Cell Rate not properly provisioned).

- Voice jitter and packet loss point to VoIP cells not being isolated from bursty data traffic, causing Cell Delay Variation (CDV) to exceed acceptable limits.

- Reduced video-conferencing quality indicates insufficient bandwidth guarantee or incorrect buffering/priority treatment for the video VC.

- Inappropriate ATM service category usage — for example, using Unspecified Bit Rate (UBR) or Available Bit Rate (ABR) for VoIP/video instead of Constant Bit Rate (CBR) or real-time Variable Bit Rate (rt-VBR) — is a direct configuration fault causing the delay and cell loss.

2. Root Cause Analysis

ATM's entire QoS model is built around correctly matching an application's traffic profile to one of five service categories: CBR, rt-VBR, nrt-VBR, ABR, and UBR, each with its own Cell Loss Priority (CLP) and traffic contract (PCR, SCR, MBS). The case explicitly states that engineers found applications using inappropriate service categories — this is the primary root cause. If VoIP (which is delay-intolerant but has a smooth, near-constant bit rate) is assigned to UBR (no guarantees, best effort) or ABR (rate is throttled dynamically based on network feedback), cells will be delayed, dropped, or arrive with high jitter whenever the switch is busy. Similarly, if video conferencing (bursty but delay-sensitive) is not assigned rt-VBR with an appropriately dimensioned Sustainable Cell Rate and Maximum Burst Size, its cells get queued behind lower-priority best-effort traffic during congestion, degrading picture quality. High switch utilization and VC congestion are secondary effects: because CBR/rt-VBR traffic is not properly reserved and isolated, the switches cannot enforce priority queuing correctly, so all traffic classes contend for the same buffers, and traffic engineering (VC routing across the physical topology) has not been optimized to route around heavily loaded links.

3. Debugging Methodology

- Retrieve per-VC statistics from the ATM switches (cell counts, discarded cells, CLP-tagged cell ratio) to identify exactly which VCs are experiencing loss and congestion.

- Cross-reference each VC's configured service category and traffic contract (PCR/SCR/CDVT) against the actual application it carries, flagging mismatches (e.g., VoIP mapped to UBR).

- Measure Cell Delay Variation (CDV) and Cell Transfer Delay (CTD) for the VoIP and video VCs during peak load using ATM test equipment or SNMP polling, and compare against ITU-T QoS class targets.

- Analyze switch CPU/buffer utilization graphs over a 24-hour period to pinpoint the exact peak windows and which physical trunks/switches are saturated.

- Review the traffic engineering/VC routing table to check whether alternate, less-congested paths exist between endpoints that are not currently being used.

4. Optimization Strategies

- Re-map every application to the correct ATM service category: assign VoIP to CBR (fixed low-bandwidth, strict delay guarantee), video conferencing to rt-VBR (bursty but delay-sensitive), online banking transaction traffic to nrt-VBR (delay-tolerant but reliable), and only bulk/background data to UBR/ABR.

- Properly dimension the traffic contract parameters (PCR, SCR, Maximum Burst Size, CDVT) for each VC based on measured application requirements, rather than using default values.

- Implement Connection Admission Control (CAC) so that new VCs are only admitted if sufficient bandwidth remains to meet existing QoS guarantees, preventing over-subscription.

- Apply traffic engineering to redistribute VCs across underutilized alternate paths, load-balancing across the ATM backbone instead of concentrating all traffic on the shortest path.

- Increase buffer and trunk capacity at switches identified as chronically over-utilized, or upgrade trunk links between the most congested switch pairs.

- Enable per-VC monitoring/alerting so that congestion or CLP-tagged cell discards trigger proactive alerts before customers are affected.

5. Analysis and Justification

Correcting the service-category mismatch is the single highest-impact fix because it directly restores ATM's native QoS guarantees for delay-sensitive traffic without any hardware change. Adding CAC prevents the same over-subscription problem from recurring as the network grows. Traffic engineering and capacity upgrades address the structural congestion at switches operating near saturation. Together, these steps restore predictable low-latency, low-jitter performance for VoIP and video, while preserving reliable throughput for banking transactions, directly resolving every symptom described in the scenario.

Question 3 — MPLS Network Debugging and Optimization

An e-commerce company's MPLS backbone connects HQ, regional offices, warehouses, and cloud data centers, supporting order processing, ERP, VoIP, video conferencing, and inventory management. Some LSPs are congested while others are underutilized, several routers have incorrect label mappings, and traffic engineering is not configured. Investigate, debug, and optimize.

Answer

1. Problem Identification

- Uneven LSP (Label Switched Path) utilization: some LSPs are heavily congested while parallel LSPs remain idle, indicating a lack of load-balancing/traffic engineering across the MPLS backbone.

- Incorrect label mappings at several routers, causing packets to be forwarded along wrong or suboptimal LSPs, directly increasing latency and potentially causing routing loops or drops.

- No configured traffic engineering (MPLS-TE), meaning LSPs are established purely based on IGP shortest path (e.g., OSPF/IS-IS cost) rather than actual available bandwidth, so multiple applications pile onto the same “shortest” path.

- Application-level symptoms — slow cloud access, delayed order processing, poor VoIP quality, and video interruption — are the downstream effects of the above LSP and label problems.

2. Root Cause Analysis

MPLS forwards packets based on labels assigned via LDP (Label Distribution Protocol) or RSVP-TE rather than repeated IP lookups at every hop. If a router's Label Forwarding Information Base (LFIB) contains an incorrect label binding — due to a misconfiguration, a stale entry after a topology change, or an LDP session failure that was not cleanly re-converged — packets can be forwarded down an unintended LSP, adding hops, delay, or even being dropped if the label has no valid entry at the next router. The uneven LSP load is a direct consequence of relying on default IGP-based path selection: standard OSPF/IS-IS metrics choose the same “best” path for all traffic regardless of the current load, so as data volume grows, that one path saturates while equally valid alternate paths sit idle. Because MPLS Traffic Engineering (RSVP-TE with constraint-based routing) was never configured, the network has no mechanism to consider available bandwidth, nor to explicitly reroute LSPs around a congested link, nor to reserve bandwidth per traffic class — explaining both the congestion and the poor QoS for VoIP/video.

3. Debugging Methodology

- On each core and edge router, verify the LDP/RSVP-TE neighbor sessions are up and check the LFIB/label bindings against the expected topology (e.g., using ‘show mpls forwarding-table’ and ‘show mpls ldp bindings’ equivalents) to locate incorrect or stale label mappings.

- Trace a sample of affected traffic flows (traceroute/MPLS LSP ping and LSP traceroute) end-to-end to confirm the actual path taken versus the expected shortest path.

- Collect per-LSP utilization statistics over a peak-hour window to quantitatively confirm which LSPs are congested and which parallel paths are underused.

- Review the IGP topology database and current traffic-engineering (or lack of) configuration to confirm that path selection is purely IGP-metric based.

- Correlate application-level complaints (slow ERP, VoIP jitter) with the specific congested LSPs identified above to confirm causation, not just correlation.

4. Optimization Strategies

- Correct and re-verify all label bindings at the affected routers; if caused by LDP session instability, tune LDP timers and enable graceful restart to prevent future label-table corruption during reconvergence.

- Deploy MPLS Traffic Engineering (RSVP-TE) with constraint-based routing so LSPs are established considering real-time available bandwidth, not just IGP cost, automatically steering new LSPs away from congested links.

- Implement explicit-path LSPs or auto-bandwidth adjustment for high-priority traffic (VoIP, video) to guarantee bandwidth and low-latency paths independent of best-effort traffic.

- Enable MPLS DiffServ-aware Traffic Engineering (DS-TE) so different traffic classes (Expedited Forwarding for VoIP, Assured Forwarding for ERP/order data, Best Effort for bulk transfers) receive separate bandwidth pools per class along the LSPs.

- Use Fast Reroute (FRR) to provide sub-50ms failover for critical LSPs in case of link/node failure, improving resilience for order-processing and VoIP traffic.

- Load-balance across equal-cost or engineered alternate LSPs to fully utilize the backbone capacity instead of concentrating traffic on a single shortest path.

5. Analysis and Justification

Fixing the label mapping errors removes the immediate forwarding faults causing latency and drops, while enabling MPLS-TE with DiffServ awareness addresses the structural root cause — the absence of bandwidth-aware path selection — by actively balancing load and reserving capacity for delay-sensitive classes. Fast Reroute adds resilience so that a single link failure does not repeat the same congestion pattern. This combination directly restores efficient utilization across the whole backbone and provides guaranteed low-latency treatment for VoIP and video, resolving the business-critical symptoms reported by the e-commerce company.

Question 4 — Automated Warehouse: Error Detection and Correction Optimization

In an automated logistics warehouse, barcode scanners, RFID readers, and robots exchange shipment and inventory data continuously. Some shipment records arrive corrupted, scans occasionally fail, and inventory counts mismatch. Increased transmission errors occur during peak hours due to electrical interference, signal attenuation, and congestion; some corrupted frames go undetected. Investigate the weaknesses in the current error control mechanism and recommend improvements.

Answer

1. Problem Identification

- Undetected corrupted frames reaching the application layer and being written to the database as incorrect records — the most serious symptom, implying the current error-detection code is too weak or improperly applied.

- Occasional scan failures, consistent with transient bit errors caused by electrical interference (from motors/conveyors/RF equipment common in warehouses) and signal attenuation over cable or wireless runs.

- Rising errors specifically during peak hours, indicating that network congestion (queuing delay, buffer overflow, collisions) is compounding the physical-layer noise problem.

- Retransmission mechanism exists (“some corrupted frames are detected and retransmitted”) but is evidently incomplete, since other corrupted frames are not caught at all.

2. Root Cause Analysis

The core weakness is very likely the use of a simple, low-strength error-detection code such as a basic parity bit or a weak checksum instead of a robust Cyclic Redundancy Check (CRC). A single parity bit can only detect an odd number of bit errors and fails silently on an even number of flipped bits — exactly the kind of undetected corruption described. Even if CRC is used, a CRC of insufficient bit-length (e.g., CRC-8 instead of CRC-32) has a higher probability of an undetected error (an error pattern that happens to produce the same remainder) especially over noisy links with high bit-error rates. Additionally, if error detection is only implemented at one layer (e.g., only at the data-link layer) without an end-to-end check at the application/transport layer, corruption introduced by intermediate processing, buffering, or a faulty NIC can bypass detection entirely. The peak-hour spike points to a compounding second factor: as offered load rises, collision/congestion-induced frame corruption and buffer-overflow drops increase, and the weak error code is even less likely to catch multi-bit burst errors that occur under electrical interference, since burst errors are precisely the pattern basic parity and short CRCs handle poorly.

3. Debugging Methodology

- Capture raw frames at the physical/data-link layer during a peak-hour window using a protocol analyzer to measure the actual bit-error rate (BER) and characterize whether errors are single-bit or burst in nature.

- Inspect the current frame format to identify exactly which error-detection scheme is implemented (parity, checksum, or CRC) and its bit-length/polynomial.

- Deliberately inject known corrupted frames (fault injection) in a test environment to empirically measure the current scheme's detection rate versus the theoretical expected undetected-error probability.

- Correlate the electrical/RF interference sources in the warehouse (motor startup transients, RFID reader emissions) with the timestamps of scan failures to confirm noise as a contributing cause.

- Review buffer/queue statistics at the central server and access points during peak load to quantify congestion-related frame loss versus corruption-related frame loss.

4. Optimization Strategies

- Replace weak parity/checksum schemes with CRC-32 (or a suitably long CRC for the frame size in use), which offers dramatically lower probability of undetected error and is well-suited to detecting the burst errors typical of electrical interference.

- Add an end-to-end checksum or hash validation at the application layer in addition to the data-link CRC, so that any corruption introduced after the initial link-layer check (e.g., in buffering or processing) is still caught before a database write occurs.

- Implement Automatic Repeat reQuest (ARQ) with sequence numbering (e.g., Selective-Repeat ARQ) so that any frame failing the CRC check is automatically retransmitted, rather than relying on inconsistent detection.

- For consistently noisy links, introduce Forward Error Correction (FEC), such as Hamming codes or Reed-Solomon codes, allowing the receiver to correct small burst errors without waiting for retransmission — particularly valuable for real-time robot-to-server communication where retransmission delay is costly.

- Mitigate the physical-layer noise itself: use shielded/twisted-pair cabling or fiber where feasible, physically separate data cabling from high-interference motors/conveyors, and, for wireless scanners, move to frequency-hopping or higher-immunity RF protocols.

- Address the congestion contribution by implementing traffic prioritization for scan/inventory update frames and increasing buffer capacity or bandwidth at the central server during peak hours.

- Add a database-level reconciliation/audit process that periodically cross-checks physical stock counts against recorded transactions to catch any residual undetected errors and flag anomalies for review.

5. Analysis and Justification

Upgrading to CRC-32 with an additional application-layer integrity check closes the specific gap that allows corrupted frames to reach the database undetected, since a strong CRC's undetected-error probability is orders of magnitude lower than a parity bit, especially for burst errors. Adding Selective-Repeat ARQ ensures that every detected error is automatically corrected via retransmission without manual intervention, while FEC further reduces the retransmission burden on time-sensitive robot communications. Addressing the physical interference sources and peak-hour congestion tackles the problem at its origin rather than only at the symptom level. Together, these layered measures (physical mitigation, stronger detection, correction via ARQ/FEC, and application-level reconciliation) provide defense-in-depth, ensuring shipment and inventory data integrity even under the warehouse's noisy and congested operating conditions.

Question 5 — Online Education Platform: Flow Control Debugging and Optimization

An online education platform serves 15,000+ students during live exams from a 10 Gbps server, while students use slower and heterogeneous broadband/mobile connections. Peak-hour symptoms include incomplete downloads, delayed page loads, duplicate packets, repeated retransmissions, receiver buffer overflow, application crashes, excessive retransmissions, increased latency, reduced throughput, and poor bandwidth utilization. Investigate the flow-control weaknesses, debug the system, and recommend optimization techniques.

Answer

1. Problem Identification

- Receiver buffer overflow and application crashes: the server is sending data faster than slower client devices/networks can consume, exceeding the receiver's buffer capacity — a classic flow-control failure (fast sender, slow receiver).

- Duplicate packets and excessive retransmissions: consistent with a sender that times out and retransmits before accounting for the true round-trip time (RTT) of slower clients, or with receivers discarding out-of-order/overflowed segments and requesting the same data again.

- Incomplete downloads and delayed page loads: symptomatic of the effective throughput being throttled well below what either end could individually support, due to repeated loss-and-retransmit cycles rather than smooth delivery.

- Poor bandwidth utilization despite a 10 Gbps server link: indicates the bottleneck is not the server's link but the control mechanism failing to adapt the sending rate to each individual client's much smaller capacity.

2. Root Cause Analysis

The scenario explicitly states that “the current flow control mechanism is not effectively regulating the transmission rate between the sender and receivers,” which is the central root cause. In a sliding-window flow-control scheme (as used by TCP), the sender should only transmit as much data as fits within the receiver's advertised window (rwnd), which reflects the receiver's actual buffer availability. If the server is using a fixed, large, or static window size uniform across all 15,000+ heterogeneous clients rather than dynamically respecting each client's advertised window, it will keep pushing data even after a slow client's buffer is full, causing overflow and forced packet drops at the receiver. Because the platform simultaneously multicasts/serves the same large content (question papers, images, videos) to all students at once, a one-size-fits-all transmission rate is fundamentally mismatched to the wide range of access-link speeds (high-speed broadband versus mobile data). The resulting packet loss triggers retransmission timers, but if Retransmission Timeout (RTO) values are not adaptively computed per-connection (i.e., not using a proper RTT-variance-based algorithm such as Jacobson's algorithm), spurious retransmissions occur even when the original packet was merely delayed, not lost — explaining the duplicate-packet symptom. This creates a compounding cycle: loss triggers retransmission, retransmission adds more load during an already-congested peak window, which causes further loss.

3. Debugging Methodology

- Capture TCP-level traces at both the server and a sample of slow and fast clients during peak load (Wireshark/tcpdump) to examine advertised receive-window sizes, retransmission events, and duplicate ACK patterns.

- Measure RTT distribution across the student population to quantify the disparity between fast broadband and slow mobile connections, and check whether the server's RTO calculation accounts for this variance.

- Analyze server-side connection logs for the number of concurrent connections and per-connection throughput during the exam window to identify whether a fixed transmission rate or window size is being applied uniformly.

- Examine receiver-side buffer configuration and application-layer read rates to determine whether slow client devices are failing to drain their receive buffers quickly enough (e.g., due to CPU-limited devices simultaneously rendering video/images).

- Review whether congestion control (e.g., TCP Cubic/BBR) is properly distinguishing between congestion-based loss and simple slow-receiver conditions, since conflating the two can cause inappropriate rate reduction or inappropriate aggressive resending.

4. Optimization Strategies

- Ensure strict adherence to sliding-window flow control with dynamic receiver-window scaling (TCP Window Scaling, RFC 1323) so the server always respects each individual client's actual advertised window rather than a fixed default, preventing buffer overflow.

- Implement adaptive/RTT-aware retransmission timeout calculation (Jacobson/Karels algorithm with RTT variance) per connection, so slow mobile clients are not penalized with premature, spurious retransmissions that create duplicate packets.

- Introduce server-side rate shaping/pacing per client tier (e.g., detect connection speed at session start and cap the initial sending rate accordingly), avoiding an initial burst that immediately overflows a slow client's buffer.

- Use adaptive bitrate/content delivery for videos and large images (similar to HTTP Adaptive Streaming) so that slower clients automatically receive a lower-bandwidth version instead of the same large payload as fast clients.

- Offload static content (question papers, images, videos) to a Content Delivery Network (CDN) or edge caching layer to reduce simultaneous load on the origin server and shorten the effective RTT for geographically distributed students.

- Apply admission control / staggered release of exam content (e.g., slight time-jitter in serving question papers) to avoid a synchronized burst from all 15,000 students hitting the server in the same instant.

- Enable selective acknowledgment (SACK) so that a single lost segment does not force retransmission of an entire window of already-received data, improving efficiency under loss.

- Monitor and dynamically tune server and OS-level socket buffer sizes to match the scaled-up concurrent connection count expected during examinations.

5. Analysis and Justification

Correct sliding-window flow control with window scaling directly prevents the receiver-buffer-overflow and crash symptoms by ensuring the server never sends more than a client can currently absorb. RTT-aware RTO calculation removes the spurious duplicate retransmissions caused by treating all clients as if they had the same network latency. Adaptive bitrate delivery and CDN offloading address the underlying mismatch between the server's 10 Gbps capacity and the students' much smaller and heterogeneous access links, ensuring bandwidth is utilized efficiently rather than wasted on repeated failed transfers. Together, these changes convert a rigid, one-size-fits-all transmission model into an adaptive system that scales gracefully to each student's actual network conditions, which is precisely what is required to eliminate incomplete downloads, buffer overflows, and excessive retransmissions during high-traffic examination periods.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient ) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Identification | 20% | Accurately identifies all errors and root causes with clear understandin g | Identifies most errors with minor gaps | Identifies some errors but misses key issues | Unable to identify errors correctly |
| Debugging | 25% | Applies systematic debugging techniques effectively to resolve issues | Uses appropriate debugging methods with minor errors | Limited debugging approach; requires guidance | Unable to debug or resolve issues |
| Optimization | 20% | Optimizes code by significantly improving time complexity using efficient algorithms | Improves time complexity with minor gaps in efficiency | Limited improvement in time complexity | No improvement; inefficient approach retained |
| Analysis | 20% | Demonstrates strong logical reasoning and clear justification of solutions | Shows reasonable analysis with some justificatio n | Limited analysis; weak reasoning | No analytical reasoning demonstrated |
| Code Quality | 15% | Produces clean, well-structured code with proper comments and documentatio n | Code is mostly clear with minor documentat ion gaps | Code lacks structure and proper documentatio n | Poorly written code with no documentatio n |