SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL5)

Assessment Tool Weightage: 40%

QUESTIONS: 3								Total Marks:30

Annexure A

Constraint-Based Problem Solving

Code of Conduct:

I S HARIPRIYA(192412448)(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

priya

# Question 1: Connectivity for a Remote Rural Branch Office

## 1.1 Understanding the Problem & Constraints

The organisation is opening a branch in a location with no wired broadband (no fibre/DSL/leased line), yet the office must run cloud applications (ERP/CRM/SaaS), HD video conferencing, and a secure link to headquarters. The problem is therefore not just 'get internet' but 'get reliable, secure, business-grade connectivity under a tight budget, with room to grow.' Four constraints bind the design: (1) bandwidth — enough throughput for simultaneous conferencing and cloud sync; (2) reliability — the business cannot tolerate long outages; (3) cost — capital and recurring spend must stay low; (4) scalability — the solution must grow as headcount and applications increase, without a redesign.

## 1.2 Problem Decomposition

- Sub-problem A — Last-mile access: how does the office reach the internet at all without wired infrastructure?

- Sub-problem B — Redundancy: what happens when the primary access method fails or degrades?

- Sub-problem C — Local network: how are the LAN, Wi-Fi and end-user devices structured inside the branch?

- Sub-problem D — Security: how is traffic to headquarters and the cloud protected in transit and at the edge?

- Sub-problem E — Cost/scale: how is the design kept cheap now while still allowing painless growth later?

## 1.3 Constraint-to-Design Mapping

| Constraint | Risk if ignored | Design response |
| --- | --- | --- |
| Bandwidth | Choppy video calls, slow cloud sync | Satellite broadband (VSAT/LEO, e.g. Starlink Business, 50–150 Mbps) as primary access |
| Reliability | Business halts during an outage | 4G/5G LTE router as automatic failover on a dual-WAN SD-WAN device |
| Cost | Budget overrun, unusable ROI | Pay-as-you-grow satellite/LTE data plans instead of a costly leased line |
| Scalability | Redesign needed as branch grows | SD-WAN + cloud-managed devices that accept extra bandwidth tiers or sites centrally |

## 1.4 Proposed Network Design

- Primary WAN link: VSAT/LEO satellite broadband — the only realistic 'always-available' option where fibre/DSL does not reach.

- Secondary WAN link: 4G/5G LTE modem/router, used as an automatic failover path and to offload light traffic during peak satellite congestion.

- Edge device: an SD-WAN capable dual-WAN router that continuously measures latency/jitter/packet loss on both links and steers video-conferencing traffic to whichever path is healthiest.

- LAN layer: one managed Layer-2 switch plus a single Wi-Fi 6 access point is sufficient for a small branch, keeping capital cost minimal.

- Power resilience: a UPS (and, if grid power is unreliable, a small solar-assisted backup) protects the router/switch/AP during outages.

## 1.5 Security Mechanisms

All traffic between the branch and headquarters is carried over a permanent site-to-site IPSec VPN tunnel, so cloud/ERP traffic never traverses the public internet unencrypted. A UTM/next-generation firewall at the branch edge provides stateful inspection, basic intrusion prevention and content filtering. Multi-factor authentication (MFA) is enforced for every cloud application login, and Wi-Fi access uses WPA3-Personal with a rotated pre-shared key (a small office does not need a full RADIUS deployment).

## 1.6 Justification (Bandwidth, Reliability, Cost, Scalability)

Satellite-plus-LTE failover directly answers the bandwidth and reliability constraints together: satellite supplies the sustained throughput needed for cloud apps and conferencing, while LTE is a cheap insurance policy against satellite dropouts (e.g., weather fade). On cost, this avoids the six-figure capital cost of trenching a leased line into a remote area — the branch instead pays predictable monthly data-plan fees. On scalability, because the SD-WAN router and VPN are software-defined, the branch can add a second LTE SIM, upgrade the satellite data tier, or connect an additional branch through the same architecture without replacing hardware — the design that is affordable on day one is also the design that scales on day one thousand.

## 1.7 Risk Mitigation & Monitoring

| Risk | Likelihood/Impact | Mitigation |
| --- | --- | --- |
| Satellite outage (weather fade) | Medium / High | Automatic SD-WAN failover to LTE within seconds; alerting to IT |
| LTE tower congestion at peak | Low / Medium | Traffic shaping prioritises VPN/conferencing packets over sync |
| Power failure at branch | Medium / High | UPS bridges short outages; solar-assisted backup for longer ones |
| Credential theft on cloud apps | Low / High | MFA mandatory on every login; VPN restricts access to known IPs |

## 1.8 Implementation Roadmap

Phase 1 (Week 1–2): install and activate the satellite terminal and LTE router, configure the SD-WAN device with failover rules, and stand up the site-to-site VPN to headquarters. Phase 2 (Week 3): deploy the LAN switch, Wi-Fi 6 AP, firewall policy and MFA enforcement, then run load tests simulating simultaneous video calls and cloud sync. Phase 3 (ongoing): monitor link quality centrally through the SD-WAN dashboard, review monthly data usage against the chosen plan tier, and revisit bandwidth/backup capacity every six months as headcount grows — keeping the branch's connectivity a scheduled review item rather than a reactive fix.

# Question 2: Campus-Wide Wireless Network for 3,000+ Students

## 2.1 Understanding the Problem & Constraints

The university must give more than 3,000 students Wi-Fi access across classrooms, laboratories, the library and hostels. The core tension is between coverage/capacity and cost: only a limited number of access points (APs) can be purchased, so each AP must be placed and configured to serve as many concurrent, interference-free users as possible, while access stays secure.

## 2.2 Problem Decomposition

- Sub-problem A — Demand mapping: how many concurrent users does each building realistically generate?

- Sub-problem B — AP placement & density: where should the limited AP budget be spent for maximum coverage per rupee?

- Sub-problem C — RF planning: which frequency bands and channels minimise interference between neighbouring APs?

- Sub-problem D — Authentication: how is each of 3,000+ users verified without a bottleneck at login?

- Sub-problem E — Management: how is the whole estate configured, monitored and load-balanced centrally?

## 2.3 Constraint-to-Design Mapping

| Constraint | Risk if ignored | Design response |
| --- | --- | --- |
| Limited AP count/cost | Dead zones or overloaded APs | Density-weighted placement: more APs only where user load is highest |
| Interference | Slow, dropped connections | Dual-band channel plan (1/6/11 on 2.4 GHz; non-overlapping UNII channels on 5 GHz) + cell splitting |
| Security | Unauthorised access, data leaks | WPA3-Enterprise with 802.1X/RADIUS tied to student directory |
| Simultaneous users | Congestion at peak hours | Wi-Fi 6 (OFDMA/MU-MIMO) APs in high-density zones + QoS |

## 2.4 AP Placement & Capacity Plan

| Zone | AP type / density | Reasoning |
| --- | --- | --- |
| Library & large labs | High-capacity Wi-Fi 6 APs, higher density | Highest simultaneous device count, long dwell time |
| Classrooms | Standard Wi-Fi 6 AP per 1–2 rooms | Predictable, scheduled load |
| Hostels | Wi-Fi 5/6 APs, lower density, corridor-mounted | Lower peak concurrency, wider coverage per AP suffices |
| Open/outdoor areas | Sparse coverage, directional antennas | Lower priority, mainly best-effort browsing |

## 2.5 Frequency Band, Channel & Authentication Strategy

Each AP operates dual-band: the 5 GHz radio (more, wider, non-overlapping channels) is preferred for dense classrooms/labs, while 2.4 GHz remains available for hostel rooms where longer range through walls matters more than raw speed. A wireless LAN controller (WLC) performs automatic channel and power assignment (ACS) campus-wide, and band-steering nudges capable devices to 5 GHz to reduce 2.4 GHz congestion. For authentication, WPA3-Enterprise with 802.1X/EAP-TLS and a RADIUS server integrated with the university directory gives every student a unique, revocable credential; a separate guest SSID on its own VLAN, protected by a captive portal, isolates visitor traffic from the academic network.

## 2.6 Justification (Coverage, Performance, Security, Cost)

Weighting AP density by actual usage (library/labs get more APs than hostel corridors) means the fixed AP budget is spent where it earns the most capacity, rather than being spread evenly and leaving high-traffic areas under-served. Structured channel reuse and 5 GHz preference keep neighbouring APs from interfering, so performance holds up even at peak hours. WPA3-Enterprise gives per-user encryption and full accountability with negligible throughput overhead, and VLAN-isolated guest access keeps unmanaged devices away from sensitive academic systems — together the design meets coverage, performance, security and budget targets simultaneously.

## 2.7 Risk Mitigation & Monitoring

| Risk | Likelihood/Impact | Mitigation |
| --- | --- | --- |
| AP overload during exam week | High / Medium | QoS + per-user caps; WLC load-balances clients across nearby APs |
| Co-channel interference | Medium / Medium | Controller-driven ACS re-plans channels as usage patterns shift |
| Rogue access points | Low / High | WLC rogue-AP detection scans and flags unauthorised devices |
| Credential sharing by students | Medium / Medium | 802.1X ties access to individual login; sessions are logged |

## 2.8 Deployment Roadmap

Phase 1: conduct a site survey/heat-map study of every building to confirm real user density before ordering hardware. Phase 2: install the wireless LAN controller and roll out APs building-by-building, starting with the library and main labs where load is highest. Phase 3: configure RADIUS/802.1X, guest VLAN and QoS policies, then run a pilot with a few hundred students before campus-wide go-live. Phase 4 (ongoing): review controller analytics each semester to spot saturated APs and rebalance the channel/power plan or add APs only where usage data justifies the added cost.

# Question 3: Secure Network Architecture for a Financial Institution

## 3.1 Understanding the Problem & Constraints

The institution must let employees work with sensitive customer data and online banking systems while meeting strict regulatory security requirements (e.g., PCI-DSS-style controls), and it must do so without materially slowing the network or blowing up operating cost. The four constraints — security, performance, compliance, budget — often pull in opposite directions, so the design has to satisfy all four rather than optimise one at the expense of the others.

## 3.2 Problem Decomposition

- Sub-problem A — Segmentation: how is the network divided so a breach in one area cannot reach customer data?

- Sub-problem B — Perimeter & internal defence: where do firewalls sit, and how many are actually needed?

- Sub-problem C — Detection: how are attacks noticed quickly without slowing live transactions?

- Sub-problem D — Identity: how is every employee/session verified appropriately for the data they touch?

- Sub-problem E — Compliance & cost: how is all of this documented/auditable without needless duplicate hardware?

## 3.3 Constraint-to-Design Mapping

| Constraint | Risk if ignored | Design response |
| --- | --- | --- |
| Security | Lateral movement after a breach | VLAN segmentation + zero-trust micro-segmentation between zones |
| Performance | Slow banking transactions | IDS in passive/mirrored mode; firewalls only at real chokepoints |
| Compliance | Regulatory penalties, audit failure | RBAC, MFA, SIEM logging mapped to PCI-DSS-style controls |
| Budget | Unaffordable duplicate hardware | Firewall clustering for HA instead of per-branch redundant stacks |

## 3.4 Network Segmentation & Zone Design

| Zone | Contains | Access rule |
| --- | --- | --- |
| DMZ | Public-facing online banking servers | Only HTTPS/API traffic in from internet; no direct path to core DB |
| Application zone | Core banking application servers | Reachable only from DMZ and internal app-tier, via firewall ACLs |
| Data zone | Customer databases, encrypted at rest | Reachable only from application zone; DBA access via jump host + MFA |
| Employee/user zone | Staff workstations | RBAC-restricted access to app zone; no direct DB access |

## 3.5 Firewalls, IDS/IPS and Authentication

A clustered pair of next-generation firewalls (active/passive for high availability) sits at the internet perimeter performing deep packet inspection and stopping known exploits before they reach the DMZ; lightweight internal firewalls/ACLs at distribution switches control east-west traffic between the zones above. An IDS (e.g., Suricata/Snort-based) is attached to a mirrored port on the core switch so it inspects copies of traffic without sitting inline — this preserves transaction latency while still catching anomalies, with alerts correlated in a SIEM for compliance evidence. Every employee session requires MFA (hardware token or OTP) plus role-based access control, so a teller and a database administrator see fundamentally different levels of access even on the same physical network.

## 3.6 Justification (Security, Performance, Compliance, Budget)

Segmenting the network into DMZ/application/data/user zones means that even if an attacker compromises one segment, they cannot move directly to customer data — satisfying the security requirement. Running the IDS passively off a mirrored port, rather than inline, keeps banking transactions fast, and clustering (instead of duplicating) firewalls delivers redundancy at a fraction of the cost of parallel, fully separate security stacks. RBAC, MFA and centralised SIEM logging give the institution the audit trail regulators require, so the same architecture that keeps attackers out is also the one that proves compliance — meeting security, performance, compliance and budget together rather than trading one off against another.

## 3.7 Risk Mitigation & Monitoring

| Risk | Likelihood/Impact | Mitigation |
| --- | --- | --- |
| Phishing/credential theft | Medium / High | MFA on every login; security-awareness training for staff |
| Lateral movement after breach | Low / High | Zone-to-zone firewall ACLs block direct east-west access |
| Insider misuse of access | Low / High | RBAC limits scope; SIEM flags anomalous access patterns |
| Undetected slow-and-low attacks | Medium / Medium | SIEM correlation across firewall/IDS/auth logs over time |

## 3.8 Implementation Roadmap

Phase 1: define the zone model (DMZ/application/data/user) and migrate existing servers into the correct VLANs. Phase 2: deploy the clustered NGFW pair at the perimeter and configure inter-zone ACLs, followed by IDS mirroring at the core switch. Phase 3: roll out MFA and RBAC for all staff, integrate authentication with the SIEM, and run a controlled penetration test to validate segmentation before go-live. Phase 4 (ongoing): scheduled quarterly compliance audits and SIEM log reviews keep the architecture aligned with evolving regulatory requirements without needing a redesign.