SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0713

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

Scenario-Based

Code of Conduct:

I __S.HARIPRIYA_ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

priyaa

Annexure A

Scenario-Based

1. A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid host IP address range for each subnet.

- Calculate the number of usable host addresses available in each subnet.

- Determine the remaining unused IP address range within the given network block.

2. A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid range of host IP addresses.

- Calculate the number of usable hosts in each subnet.

- Identify the unused IP address range remaining within the allocated IP block.

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

- Convert the given IPv6 address into its compressed notation.

- Expand the compressed address back into its full hexadecimal representation.

- Identify the network prefix using the /64 prefix length.

- Calculate the total number of host addresses supported within the subnet.

- the total number of possible host addresses available within this network.

4. An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

- The destination subnet to which the packet belongs.

- The matching routing table entry.

- The next-hop network/interface used to forward the packet.

- The default route that should be used if the destination does not match any routing table entry.

5. A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each LAN.

- Identify the valid host IP address range.

- Suggest suitable IP addresses for three computers in each LAN.

- Identify the default gateway that hosts in each LAN should use for communication.

# Question 1: Smart Campus Subnetting (192.168.100.0/24)

## Question

A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned: School of Computing → /25, School of Electronics → /26, and School of Mechanical Engineering → /27. As the network engineer, design the subnet allocation by (1) determining the network and broadcast address for each department, (2) identifying the valid host IP address range for each subnet, (3) calculating the number of usable host addresses in each subnet, and (4) determining the remaining unused IP address range within the given block.

## Answer

## 1. Understanding of the Scenario

The campus has been given a single Class C private address block, 192.168.100.0/24, which contains a total of 256 addresses (192.168.100.0 to 192.168.100.255) of which 254 are usable if the block is left unsubnetted. However, keeping all departments in one flat broadcast domain is undesirable in a Smart Campus, because a single broadcast storm, ARP flood, or malware outbreak in one school would immediately affect every other department. Therefore the administrator applies Variable Length Subnet Masking (VLSM), where each department receives a subnet whose size matches its actual device count. VLSM avoids address wastage, creates separate broadcast domains, allows department-wise access-control lists on the Layer-3 device, and simplifies troubleshooting because each department is identifiable from its address range.

## 2. Identification of Key Issues

- The subnets must be allocated in decreasing order of size (/25 first, then /26, then /27) so that the blocks remain contiguous and no overlap occurs.

- Each subnet loses two addresses – the network address (all host bits 0) and the directed broadcast address (all host bits 1).

- A further address in each subnet is normally reserved for the router interface, i.e. the default gateway of that department.

- The leftover address space must be documented so that it can be used for future expansion, guest Wi-Fi, IoT sensors or point-to-point links.

## 3. Step-by-Step Calculation

Block size (magic number) = 256 − (last octet of subnet mask).

- /25 → mask 255.255.255.128 → block size 128 → 2^7 = 128 addresses, 126 usable.

- /26 → mask 255.255.255.192 → block size 64 → 2^6 = 64 addresses, 62 usable.

- /27 → mask 255.255.255.224 → block size 32 → 2^5 = 32 addresses, 30 usable.

Starting at 192.168.100.0, the School of Computing occupies 0–127; the next free boundary is 128, so the School of Electronics occupies 128–191; the next free boundary is 192, so the School of Mechanical Engineering occupies 192–223. Everything from 224 onwards remains free.

## 4. Subnet Allocation Table

| Department | Subnet / Mask | Network Address | Broadcast Address | Valid Host Range | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| School of Computing | 192.168.100.0/25 (255.255.255.128) | 192.168.100.0 | 192.168.100.127 | 192.168.100.1 – 192.168.100.126 | 126 |
| School of Electronics | 192.168.100.128/26 (255.255.255.192) | 192.168.100.128 | 192.168.100.191 | 192.168.100.129 – 192.168.100.190 | 62 |
| School of Mechanical Engg. | 192.168.100.192/27 (255.255.255.224) | 192.168.100.192 | 192.168.100.223 | 192.168.100.193 – 192.168.100.222 | 30 |

## 5. Remaining Unused Address Space

Addresses 192.168.100.224 through 192.168.100.255 (32 addresses in total) are still unallocated. This free space is exactly one /27 block, 192.168.100.224/27, whose broadcast address is 192.168.100.255 and whose usable range is 192.168.100.225 – 192.168.100.254 (30 usable hosts). It may be further divided, for example into two /28 subnets (224/28 and 240/28) for a server VLAN and a management VLAN, or into /30 subnets for router-to-router point-to-point links, each of which needs only two usable addresses.

## 6. Decision and Justification

The proposed allocation is contiguous, non-overlapping and fully summarisable back to 192.168.100.0/24 at the campus core router, which keeps the ISP-facing routing table small. Recommended gateway addresses are 192.168.100.1, 192.168.100.129 and 192.168.100.193 respectively, with DHCP pools starting a few addresses later so that switches, access points, printers and servers can be statically addressed at the bottom of each range. Inter-VLAN routing with ACLs on the Layer-3 switch will then enforce the security policy between the three schools.

# Question 2: Corporate Headquarters Subnetting (10.10.0.0/16)

## Question

A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees, and the administrator has allocated the following masks: Software Development → /24, Quality Assurance → /25, Human Resources → /26 and Executive Management → /27. Determine the network and broadcast address for each department, the valid host ranges, the number of usable hosts, and the unused address space remaining in the allocated block.

## Answer

## 1. Understanding of the Scenario

10.10.0.0/16 is taken from the RFC 1918 Class A private range and provides 65,536 addresses (10.10.0.0 – 10.10.255.255). Because the departments differ greatly in size — Software Development is the largest team while Executive Management is very small — a single fixed-length mask would waste thousands of addresses. VLSM is therefore applied again, allocating the largest subnet first and moving sequentially through the address space so that the blocks are contiguous and can later be summarised.

## 2. Key Issues Identified

- Subnetting is performed inside the third and fourth octets, so the third octet must be tracked carefully when a /24 boundary is crossed.

- Allocation must proceed largest-to-smallest to prevent fragmentation and overlap.

- Two addresses per subnet are unusable (network and broadcast); one more is conventionally reserved for the gateway.

- Executive Management, although small, should be isolated with strict ACLs because it carries confidential data.

## 3. Step-by-Step Calculation

- Software Development /24 → 256 addresses → starts at 10.10.0.0, ends at 10.10.0.255.

- Quality Assurance /25 → 128 addresses → next free boundary 10.10.1.0, ends at 10.10.1.127.

- Human Resources /26 → 64 addresses → next free boundary 10.10.1.128, ends at 10.10.1.191.

- Executive Management /27 → 32 addresses → next free boundary 10.10.1.192, ends at 10.10.1.223.

## 4. Subnet Allocation Table

| Department | Subnet / Mask | Network | Broadcast | Valid Host Range | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| Software Development | 10.10.0.0/24 (255.255.255.0) | 10.10.0.0 | 10.10.0.255 | 10.10.0.1 – 10.10.0.254 | 254 |
| Quality Assurance | 10.10.1.0/25 (255.255.255.128) | 10.10.1.0 | 10.10.1.127 | 10.10.1.1 – 10.10.1.126 | 126 |
| Human Resources | 10.10.1.128/26 (255.255.255.192) | 10.10.1.128 | 10.10.1.191 | 10.10.1.129 – 10.10.1.190 | 62 |
| Executive Management | 10.10.1.192/27 (255.255.255.224) | 10.10.1.192 | 10.10.1.223 | 10.10.1.193 – 10.10.1.222 | 30 |

## 5. Unused Address Range

Only 480 addresses out of 65,536 have been consumed. The remaining free space begins at 10.10.1.224 and continues to the end of the block at 10.10.255.255. In detail, 10.10.1.224 – 10.10.1.255 forms one free /27 block (10.10.1.224/27, 30 usable hosts), and the whole of 10.10.2.0 – 10.10.255.255 (254 complete /24 networks, i.e. 65,024 addresses) is untouched. This large reserve is excellent for future growth: new branch VLANs, a DMZ, server farms, VPN pools, wireless guest networks and /30 or /31 links between routers can all be carved from it without renumbering.

## 6. Decision and Justification

Assigning 10.10.0.1, 10.10.1.1, 10.10.1.129 and 10.10.1.193 as the respective default gateways gives a uniform, predictable addressing convention. Because all four departments fall inside 10.10.0.0/23, the core router can advertise a single summary route to the rest of the enterprise, reducing routing-table size and convergence time. Reserving whole /24 boundaries for future departments keeps the design scalable and easy to document.

# Question 3: Smart Hospital IPv6 Migration (2001:0db8:abcd::/64)

## Question

A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network 2001:0db8:abcd:0000:0000:0000:0000:0000/64. Convert the address to compressed notation, expand it back to its full hexadecimal representation, identify the network prefix using the /64 prefix length, and calculate the total number of host addresses supported within the subnet.

## Answer

## 1. Understanding of the Scenario

IPv6 uses 128-bit addresses written as eight groups of four hexadecimal digits separated by colons. A hospital that deploys infusion pumps, patient monitors, RFID tags, imaging equipment and environmental IoT sensors can easily exhaust an IPv4 subnet, whereas a single IPv6 /64 provides an effectively unlimited number of interface addresses. A /64 is also the standard subnet size required by Stateless Address Autoconfiguration (SLAAC) and by the EUI-64 interface identifier mechanism, which lets thousands of medical devices self-configure without a DHCP server.

## 2. Compression Rules Applied

- Rule 1 – leading zeros in any 16-bit group may be omitted: 0db8 becomes db8, 0000 becomes 0.

- Rule 2 – one and only one run of consecutive all-zero groups may be replaced by a double colon (::).

## 3. Solutions

| Task | Result |
| --- | --- |
| (i) Compressed notation | 2001:db8:abcd::/64 (address: 2001:db8:abcd::) |
| (ii) Full / expanded form | 2001:0db8:abcd:0000:0000:0000:0000:0000 |
| (iii) Network prefix (/64) | 2001:0db8:abcd:0000::/64 — the first 64 bits 2001:0db8:abcd:0000 form the network portion |
| (iv) Interface ID (host) bits | 128 − 64 = 64 bits |
| (v) Total addresses in subnet | 2^64 = 18,446,744,073,709,551,616 |

## 4. Explanation of the Calculation

With a /64 prefix length, the leftmost 64 bits (2001:0db8:abcd:0000) are fixed and identify the hospital subnet, while the rightmost 64 bits form the interface identifier and may vary. The total address count is therefore 2^64 = 18,446,744,073,709,551,616 addresses, which is approximately 1.8 × 10^19. Unlike IPv4, IPv6 has no broadcast address, so no address is lost to broadcast; the all-zeros interface identifier (2001:db8:abcd::) is reserved as the Subnet-Router anycast address, so in strict terms 2^64 − 1 addresses are assignable to interfaces. For practical purposes the subnet is treated as inexhaustible.

## 5. Decision and Justification

The hospital should keep the /64 boundary for every VLAN rather than subnetting further, because SLAAC and Neighbor Discovery assume a 64-bit interface identifier. Additional /64 subnets (for example 2001:db8:abcd:1::/64 for the ICU, :2::/64 for radiology and :3::/64 for administration) can be taken from the parent /48. A dual-stack migration strategy is recommended so that legacy IPv4-only medical equipment continues to function while new devices operate natively over IPv6, and privacy extensions (RFC 4941) should be enabled where patient-linked devices must not be trackable by their MAC-derived address.

# Question 4: ISP Core Router Packet Forwarding Decision

## Question

An Internet Service Provider connects multiple customer networks through a core router whose routing table contains the entries 192.168.1.0/24, 192.168.2.0/24 and 192.168.3.0/24. A packet arrives with the destination IP address 192.168.2.45. Determine the destination subnet, the matching routing table entry, the next-hop network/interface used to forward the packet, and the default route that should be used if the destination matches no entry.

## Answer

## 1. Understanding of the Scenario

A router is a store-and-forward device that makes an independent forwarding decision for every packet. When a packet arrives, the router extracts the destination IP address from the IPv4 header, compares it against every entry in the routing table by performing a bitwise AND with each entry's subnet mask, and forwards the packet out of the interface belonging to the most specific (longest prefix) match. This is called the longest-prefix-match rule.

## 2. Step-by-Step Determination

Destination address: 192.168.2.45. Applying the /24 mask 255.255.255.0 gives 192.168.2.45 AND 255.255.255.0 = 192.168.2.0. This result matches the second routing table entry exactly.

| Item | Determination |
| --- | --- |
| (i) Destination subnet | 192.168.2.0/24 (network 192.168.2.0, broadcast 192.168.2.255, valid hosts 192.168.2.1 – 192.168.2.254) |
| (ii) Matching routing entry | 192.168.2.0/24 — the only entry whose network portion matches after the AND operation |
| (iii) Next hop / egress interface | The interface (or next-hop router IP) associated with the 192.168.2.0/24 entry, i.e. the link toward Customer-B's network; if the network is directly connected, the router performs ARP for 192.168.2.45 and forwards the frame directly |
| (iv) Default route if no match | 0.0.0.0/0 (mask 0.0.0.0), the gateway of last resort pointing to the upstream ISP router |

## 3. Why the Other Entries Do Not Match

- 192.168.1.0/24 – the third octet 1 does not equal 2, so the AND result 192.168.2.0 differs from the entry.

- 192.168.3.0/24 – likewise the third octet 3 does not match.

- Only 192.168.2.0/24 matches, and because there is exactly one match, longest-prefix-match resolves immediately to that route.

## 4. Role of the Default Route

The default route 0.0.0.0/0 has a prefix length of zero, so it matches every possible destination address but is always the least specific entry. It is therefore selected only when no other entry matches, which makes it the gateway of last resort. On a Cisco device it is configured as 'ip route 0.0.0.0 0.0.0.0 <next-hop-IP>'. Without a default route the router would silently drop unmatched packets and return an ICMP 'destination network unreachable' message to the source.

## 5. Decision and Justification

The packet destined for 192.168.2.45 is forwarded out of the interface associated with 192.168.2.0/24. The router decrements the TTL field, recalculates the header checksum, rewrites the Layer-2 source and destination MAC addresses and transmits the frame. Any packet whose destination is outside the three customer networks — for example 8.8.8.8 — is forwarded using the 0.0.0.0/0 default route toward the ISP's upstream provider, which keeps the core routing table small and the forwarding process fast.

# Question 5: Two-Department LAN Design with a Layer-3 Router

## Question

A company establishes two office departments connected through a Layer-3 router. The Administration LAN is 192.168.10.0/26 and the Finance LAN is 192.168.10.64/26. The router interfaces are configured with the Administration Gateway 192.168.10.1 and the Finance Gateway 192.168.10.65. Determine the network and broadcast address for each LAN, identify the valid host IP address range, suggest suitable IP addresses for three computers in each LAN, and identify the default gateway that hosts in each LAN should use.

## Answer

## 1. Understanding of the Scenario

Both departments are carved from the same 192.168.10.0/24 block using a /26 mask (255.255.255.192), which creates blocks of 64 addresses each. Because the two LANs are separate broadcast domains, hosts in Administration cannot reach hosts in Finance at Layer 2; all inter-department traffic must be routed through the Layer-3 device, which is exactly what allows the company to apply access-control lists and monitor traffic between the two departments.

## 2. Key Issues Identified

- Block size for /26 is 64, so the subnet boundaries fall at .0, .64, .128 and .192.

- Each /26 provides 2^6 = 64 addresses, of which 62 are usable after removing the network and broadcast addresses.

- One usable address in each subnet is consumed by the router interface, leaving 61 addresses for end devices.

- Every host must be configured with the correct IP, the mask 255.255.255.192, and the gateway of its own subnet — a wrong mask is the most common cause of inter-VLAN failures.

## 3. Network and Broadcast Addresses

| LAN | Subnet / Mask | Network Address | Broadcast Address | Valid Host Range | Hosts | Default Gateway |
| --- | --- | --- | --- | --- | --- | --- |
| Administration | 192.168.10.0/26 (255.255.255.192) | 192.168.10.0 | 192.168.10.63 | 192.168.10.1 – 192.168.10.62 | 62 | 192.168.10.1 |
| Finance | 192.168.10.64/26 (255.255.255.192) | 192.168.10.64 | 192.168.10.127 | 192.168.10.65 – 192.168.10.126 | 62 | 192.168.10.65 |

## 4. Suggested Host Addressing Plan

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Admin PC-1 | 192.168.10.10 | 255.255.255.192 | 192.168.10.1 |
| Admin PC-2 | 192.168.10.11 | 255.255.255.192 | 192.168.10.1 |
| Admin PC-3 | 192.168.10.12 | 255.255.255.192 | 192.168.10.1 |
| Finance PC-1 | 192.168.10.70 | 255.255.255.192 | 192.168.10.65 |
| Finance PC-2 | 192.168.10.71 | 255.255.255.192 | 192.168.10.65 |
| Finance PC-3 | 192.168.10.72 | 255.255.255.192 | 192.168.10.65 |

The addresses .2 – .9 and .66 – .69 are intentionally left free for switches, printers, access points and servers that require static addresses, while a DHCP pool can be configured from .20 – .62 and .80 – .126 for user devices.

## 5. Default Gateway and Inter-LAN Communication

Hosts in the Administration LAN must use 192.168.10.1 as their default gateway, and hosts in the Finance LAN must use 192.168.10.65. When Admin PC-1 (192.168.10.10) sends a packet to Finance PC-1 (192.168.10.70), it ANDs the destination with its own mask, obtains 192.168.10.64, sees that this differs from its own network 192.168.10.0, and therefore forwards the frame to its default gateway. The router receives the frame, consults its routing table, finds 192.168.10.64/26 as a directly connected network on its second interface, rewrites the Layer-2 header and delivers the packet to Finance PC-1.

## 6. Decision and Justification

This design gives each department an equal, adequately sized address space, keeps broadcast domains small, and permits security policies such as 'Administration may access the Finance file server on port 445 but not the accounting database'. Verification can be performed in Cisco Packet Tracer using ping between hosts of the same LAN (Layer-2 success), ping across LANs (routing success) and the commands 'show ip interface brief' and 'show ip route' on the router to confirm that both /26 networks appear as directly connected routes.

# Conclusion

Across the five scenarios, the same fundamental principles of IP addressing were applied: identifying the block size from the prefix length, allocating subnets in decreasing order of size to avoid fragmentation, reserving the network and broadcast addresses, assigning the first usable address as the default gateway, and documenting the unused space for future growth. The IPv6 scenario demonstrated how a single /64 prefix removes the address scarcity that forces careful IPv4 subnetting, while the routing scenario showed how the longest-prefix-match rule and the 0.0.0.0/0 default route together guarantee that every packet has a forwarding decision. Together these tasks fulfil Course Outcome CO2 by configuring IP addressing schemes, subnetting and basic routing for efficient network design and topology.

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understanding of the scenario and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Misinterprets or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |